Development of

Piping & Instrumentation Diagram

(Detail Engineering Phase - Process Units)

By: A. Tabatabaei Javid

Feb 2026

Table of Content

1.	What to Do Before the First Issue of P&ID	4

2.	Drafting	5

3.	Vent	6

4.	Drainage and Drain Connections	9

5.	Equipment Details	14

5.1.	Centrifugal Pumps	14

5.2.	Metering Pumps	19

5.3.	Steam Turbine	19

5.4.	Compressors	24

5.5.	Shell & Tube Exchangers	26

5.6.	Water Coolers	27

5.7.	Air-coolers	28

5.8.	Fired Heaters	29

5.9.	Drums	31

5.10.	Storage Tanks	32

5.11.	Cooling Tower	33

6.	Equipment Trim Number	33

7.	Equipment Elevation	33

8.	Isolation Philosophy and Removable Spool Pieces	38

8.1.	Isolation at battery Limit:	38

8.2.	Isolation for on-stream maintenance	39

8.3.	Isolation for maintenance during plant shut-down.	39

9.	Emergency Isolation	43

10.	Warm-up/Start-up	43

11.	Liquid Seals for Vapor Condensation or Level Control	47

12.	Details of Injection and Sampling Point (for analyzer)	48

14.	Full Symmetrical Piping	49

15.	Reducers	49

16.	Check Valves or Non-Return Valves (NRV)	50

18.	Line Finishing	55

19.	Heat Tracing	55

20.	Strainers and Filters	59

21.	Connection to/from Top of Line	60

22.	Minimum Required Distances & Distances to be minimized	61

23.	Utility Distribution Diagrams (UDD/UHD)	68

24.	Locking Device and Car Seal Device	71

25.	Steam Trap and Condensate Pot	72

26.	Pipe Specification Break	75

27.	Insulation Kit and Insulation Flange	81

28.	Steam Condensate Drip Legs	81

29.	Line Size	83

30.	Control and Instrumentation Tip Points	87

31.	Pressure Safety/Relief Valves	96

32.	Flowmeters	98

33.	Smart Software for Preparing P&ID (e.g., AVEVA)	98

34.	Hold Management	98

35.	Pickling	99

36.	Miscellaneous	100

38.	Typical Check List	101

# What to Do Before the First Issue of P&ID

  - Ask draftsman to review the AutoCAD files from drafting point of view e.g., size of drawing, font of texts, layers, etc. Ensure the CAD files, on which the draftsman is working, are the last files.

  - Ensure that P&ID is consistent with PFD

  - Review the title block. Check the following items:

- Project Title (According to contract)

- Owner Document Number

- Contractor Document Number

- Unit Name shall be specified on P&ID. Do not expect the user to guess the section only by document number.

- Section Description

- Logo(s)

- Revision: Revision number shall be zero. Do not continue basic revision number because in case the licensor (or basic designer) revises the document, revision numbers will be duplicated.

- It is recommended to allocate different document number for each individual sheet of P&ID.

  - Consider enough room for development during next stages.

  - Review the contract

- Look for items, facilities and packages which may not be covered in process data book or basic package e.g.

  - Steam-condensate recovery

  - Flare and Flare Network (KO drum, Pumps, ...)

  - Fuel gas KO drum

  - Pumps (and sump) for transferring surface water

  - Boil-Off Gas Recovery

  - Diesel Tank

  - Storage Facilities

  - Review all equipment data sheets (especially the notes) and duty specifications

- Look for notes and data which affect the P&ID e.g., elevation, required utilities, effluents, interlock with other packages, etc.

  - Review and finalize drainage philosophy

  - Prepare the HOLD list for each individual sheet of P&ID (Refer to “Hold Management” section)

- Size of all lines to/from OSBL shall be checked and finalized. Oversize if in doubt or make it HOLD. This is to avoid any claim from off-site contractor.

  - Finalize the vendor scope of supply for long delivery equipment and package. Remember this should be a concern of departments or parties who are involved in procurement. So, as long as process data for an item, instrument, etc. are available, process department has no concern whether the item/instrument is in package vendor scope of supply or is purchased by piping, instrument department. If the process data are pending to package vendor, the item/instrument may be included in the scope of package vendor. Some examples are listed below:

- Steam traps on condensate lines out of package

- Instruments on utilities to package (Flowmeter, control valve)

- Instruments on effluents out of package (Flowmeter, control valve)

- Strainer at inlet lines to package

  - Ensure the equipment elevation and “Slope” or “No pocket” requirements are all specified on P&IDs (Refer to “Equipment Elevation” section).

  - Review the Symbol and Legend; rethink about the role of symbol and legend.

  - For complicated packages (including Compressors), the detailed P&ID supplied by vendor shall be referred to in the P&ID prepared by contractor. Tie-ins and streams or signals to/from the package shall be shown on the main P&IDs.

# Drafting

  - Symbol and Legend: Details of the followings should be shown on symbol and legend:

- Vent and drain of level instruments

- Sealing facilities of Pumps

- Details of sample connection including required utility (Water) and flare connection

  - Consider equipment index as the first sheet. It is not necessary to consider equipment List on each individual sheet if equipment index is provided (see previous item).

  - Provide a battery limit summary sheet for process lines.

  - No “Cross” is allowed.

  - All reducers shall be noted with the two relevant nominal line sizes as: large size  small size irrespective of direction of flow.

  - The purpose of all connections (e.g., purging) with valves shall be noted when not connected to a line.

It is also recommended to clarify the purpose of the lines which are normally no flow (e.g., recycling during start-up, regeneration, etc.), if the purpose cannot be understood at a glance.

  - Range of level gauge is more than transmitter for vessels/columns.

  - Revision cloud shall be considered for any change with technical impacts.

Consider different layers for each revision clouds of each individual revision i.e., there should be no need to delete the revision clouds but turning off the layer. It is not necessary to indicate the revision next to the revision clouds.

  - Flow direction should be clearly marked with arrows but do not clutter.

  - The preferred drawing size is A1.

# Vent

  - Cap / Plug or Flange

- Generally, this a piping issue

- Design temperature

- Cases which BW fittings are used in small lines (<2”)

- Service

  - Vent and drain connection of equipment should be blinded

Blind flange is safer in corrosive services especially for toxic services. Also, using threaded connection is not allowed in some services i.e., blind flange shall be used for any size (e.g., butadiene, styrene, etc.)

- Size of connection

  - Use cap or plug for small sizes (≤ 1 ½”)

  - Use blind for larger sizes

A. Plug (< 2”)

B. Cap (< 2”)

C. Blind Flange and Piece Pipe (< 2”). Recommended for corrosive or lethal services

D. Blind Flange (≥ 2")

Usually, valves with nominal size of 11/2” or smaller have welded endings. So, in case blind flange is required for blind such valves, a piece pipe and additional flange is required.

  - Most often, air-coolers do not need vent line (or header) as they may be vented via the upstream equipment. Only cap/blind is enough for each bundle. In case each bundle/bay is isolable, the venting can be done via the drain header line.

The exception to this recommendation is where there the risk of accumulation of non-condensable is credible during normal operation.

  - Vent lines should be no pocket.

  - Vent of pumps should be sent to the upstream drum. In vacuum services (e.g., pumps handling cold condensate from surface condensers or pumps at bottom of a vacuum tower) and highly volatile liquids (e.g., LPG), it is a must (See API RP 686-2009; §3.1.1.2). The vent line shall be minimum ¾”.

Note: In the latter case (highly volatile services), a connection to safe location (should be considered so that the pump can be depressurized for maintenance.

  - Water Coolers (Vent and Drain)

- Consider one vent at high point of cooling water return line

- Consider two drains at low points of supply and return branches. If four way back-flush valve is used and the cooling water headers are underground, drains should be between isolating valves and four-way valve. See also § Water Coolers.

  - In case a heat exchanger is isolable, evaluate the need for utility connection, vent and drain to safe location if the fluid is volatile, flammable or lethal.

  - Drums (Vent and Ventilation): Refer to IPS-E-PR-200 for size and necessity.

- It is not acceptable to locate vent connection of vessel or tower on the connecting line.

  - Overflow nozzle for atmospheric tanks and tanks/vessels with blanketing

# Drainage and Drain Connections

  - Make Drainage Philosophy Clear. Licensors may not take care of it.

  - Capped / Plug or Flanged (Size, Corrosion and Service): Same as vent connection. Drain of vessels and towers are most often blinded. Note that “operating drain” connections need neither cap/plug nor blind flange.

  - If it is not clear in the project criteria whether the drain size is mentioned for open drain or closed drain, consider the mentioned size for open drain. Size of the above ground closed drain branches may be the lesser of 2 inches or open drain size for all equipment.

  - Specify the services and the volume in which all low point drains which are provided for hydrotest shall be equipped with valve.

  - Drain for Hot condensate Lines and BFW: One connection for plant water may be considered to avoid steam formation during draining of the line.

  - Ensure proper draining facility (e.g., funnel) is available at Valve Station.

  - Oily Sewer, Non-Oily Sewer and Closed Drains:

- Non oily sewer: The system shall collect special oil free waters containing high total dissolved solids such as:

  - Boiler blow-down

  - Desalination unit blow-down

  - Neutralized effluents from all neutralization sumps through the plant such as DM water production unit (or polishing).

  - Storm, fire and washing run-off waters from sulphur solidification and crushing area after removal of sulphur particles through a sedimentation sump.

  - Cooling water (circulated) blow-down and drains, provided that there is no possibility of oil contamination. Mind the corrosion inhibitor (phosphate) if the make-up water is desalinated water.

  - Non-oily water sewer is disposed to either evaporation ponds or sea. Sometimes, this may be required to be treated and reused in the plant.

- Provision for draining control sets, exchanger, etc. via hose connection on closed drain arrangement. Watch the fluid properties (Toxic/Lethal, Auto-Ignition Point, Flash Point)

  - Be careful of siphon effect for draining of lines to vessels/tanks with dip pipe.

  - Vessels with both oily and closed drain:

  - Provide a drain with valve for all columns and vessels. Generally, this valve should be located on the bottom line outside the skirt and downstream of the fire-resistant S/D Valve, if available.  The drain valve should be located on the bottom of the vessel when:

- No bottom line is present, or

- The bottom line is not flush with the lowest point of the vessel.

  - Draining to Flare (Vapor Pressure ≥ 34.5 kpaa):

Equipment containing volatile hydrocarbon need a “drain to flare” connection.

If the drain will be used during maintenance and the equipment can be connected to flare header via another line e.g., PSV by-pass, there is no need to consider a dedicated drain to flare connection.

If drain may be used while the equipment is in service, (e.g., fuel gas knock-out drum), other connections to flare may not serve as the “drain to flare” connection. Hence, dedicated “drain to flare” connection should be considered.

  - Generally, sight glass and check valve is not required on closed drain line unless for some operational drains.

  - Review if the closed drain system may be exposed to low temperatures.

  - Ensure that both sides can be drained:

- Tower or vessel with baffle (Check also for steam-out connection)

- Kettle Reboiler

  - In case there is a pot inside a vessel, provide drain facilities.

  - Size of Vent/Drain (and all other nozzles) for Hydrogen service, HP steam and lethal services should be minimum 1 inch.

  - Minimum Size of vent, drain and utility connections for highly viscous fluids (e.g., atmospheric/vacuum residue) should be 1 inch.

  - Spool may be required at pump suction/discharge (for impeller/shaft removal). See API RP 686 (2009); § 2.10.

  - No funnel is mandatory for TSVs on water services. Water may be routed to ground.

  - No funnel is mandatory for Heat Exchangers. Equipment may be drained using hoses.

  - Draining of level instrument (Stand pipe, level gauge, level transmitter with or without diaphragm seal): Check if it is not necessary to consider permanent piping for draining of level instruments.

  - Most often, air-coolers do not need drain lines as they van be drained via upstream/downstream equipment. Only one valve with cap/blind per bundle is enough.

An exception to this rule is where each bundle/bay can be isolated for maintenance while the unit is in operation. If this is the case, and if the service is flammable, toxic or lethal, bundles drains will be connected to a drain header line which will be sent to flare or closed drain. Depressurizing of the bundles may be done via this drain header line. No additional vent header is necessary. For such cases, evaluate if a utility connection is necessary on each bundle for purging.

  - It is recommended to not combine vent/drain lines of pumps. This is because:

- Their size is usually small and hence they are welded. And

- To dismantle pump casing, it is necessary to remove vent/drain connection.

- Take care of vent lines from Seal pots (API Plan 52): Individual flare connection is necessary. Do not merge the vent of seal drums with other lines. Using individual flare connection is also applicable to Compressor dry gas seal vent and distance piece of reciprocating compressors handling toxic gasses.

The issue is generally a piping issue.

  - It is recommended to consider a sweep gas connection for closed drain drum.

  - Steam ring should be considered for purging of sump.

  - It is preferred to not bury the closed drain drum. Buried drums need higher thickness and are more difficult for inspection.

  - There should be a curb around the closed drain sump to prevent surface water entering the sump (minimum 100 mm).

  - Provide drains on utility lines to buildings. The drain shall be close to the building. This drain will be used for line flushing during start-up and maintenance.

  - Insulation flange shall be installed upstream of the isolation valve of the non-buried closed drain drums.

  - Bypass line of the control valve should be installed at the same level of the control or higher so that the bypass line needs no drain. Otherwise, a drain connection should be provided.

3/4”

3/4”

Sometimes a double block is preferred on vent and drain connections at high ratings (e.g., 600# and higher). Note that where it is possible to vent the system before draining, the double block is not necessary for drain connections.

  - Recommended Size of Vent and Drain for Pipes at Battery Limit

| Line Size | Size of Vent and Drain |
| --- | --- |
| For Line Size ≤ 3” | 1” |
| For Line Size between 4” to 10“(Inclusive) | 2” |
| For Line Sizes ≥12” | 3” |

# Equipment Details

The minimum information to be shown on general P&IDs should be as follows. Vendor P&IDs include much more details not specified here.

## Centrifugal Pumps

- Size of nozzles

- Driver

- Consider foot valve and priming line (or connection) for suction lift pumps if self-prime pump is not selected.

- Vent and drains

According to API RP 686:

2.10.2 Piping vents and drains shall be located in breakout spools on the inlet and outlet piping to the machine.

- Connect vent of pipe and casing to upstream vessel where practical.

- Size of block valves (NIOEC Concept)

- Check the necessity of PSV for LPG pumps, if can be isolated.

- For highly volatile liquids (e.g., LPG), it is recommended to provide N2 purge connection. N2 may be supplied from utility stations. Other connections for temporary purging, if considered around the pump, are acceptable.

Also, N2 purge-out connection should be provided where the system shall be maintained free from moisture (e.g., cryogenic or pyrophoric service).

- External flushing, if any

  - Select flush fluid not likely to vaporize or degrade the product. Flow rate is usually ≤ 1 m3/hr.

  - Set pressure is usually maximum suction pressure +2.0 kg/cm2G. PI indicates whether strainer chokes.

- Seal System for dual seal pumps

  - Utilities as required (e.g., cooling water)

  - Gages, switches or transmitters

- Instrumentation

  - It is good to have a differential pressure transmitter (with DPAH) around strainers of pumps with pressure rise (ΔP>35 bar). A utility/purge connection at upstream of the strainer may also be recommended for lethal or very toxic services.

  - Local Start/Stop: The local start/stop button of motors usually do not act via the control system. Hence, it is not shown on P&ID.

  - Remote start/stop

  - Stand-by auto-start (Flow, Pressure, etc.)

    - Fired reboiler circulating pumps

    - Steam generation (e.g., HRSG) circulating pump

    - Surface condenser condensate pumps

  - Status (Run/Stop)

  - MCC Fault

  - DCS or safety interlocks which start/stop pump

- Centrifugal pumps have usually check valve(s) and a pressure gauge upstream of the check valve(s) on the discharge line.

- In rare cases, the pump received the fluid from discharge of another pump (i.e., there is no drum upstream of the pump). In such cases, using a PT (with PAL) at upstream of the pumps is recommended.

- Check the necessity of PSV at the pump discharge, if the pump is driven by turbine or diesel engine.

Basic Arrangement for Centrifugal Pumps

Flushing Connection

- Minimum Flow Protection:

  - Definition

  - Destination:

    - In case there is an upstream drum/tank, the minimum flow line should be connected to the upstream drum.

    - If the inlet stream to the upstream drum/tank is liquid phase, it is recommended to connect the minimum flow to the main inlet stream line to the drum.

    - If it is not practical to send the minimum flow to the upstream drum/tank, the minimum flow line may be connected to the common suction line of the pumps.

  - Methods of Protection

    - Pressure Control Loop (Beware of flat H-Q curves)

      - Note: For parallel operating pumps (i.e., M+N arrangement where M ≥ 2), pressure control loop may be successfully used and FIC loop should not be used unless FIC is provided for each pump). In this case, it is recommended to provide a PT (with PAH) at downstream of each pump (upstream of the discharge block valve). The respective PIs, high selector (or low selector depending on the control scheme) and PIC will protect the pumps against the minimum flow.

    - Flow Control Loop with one Flow Transmitter (Less Convenient)

Attention: In any case, the flow transmitter shall be installed upstream of min flow protection branch.

    - Flow Control Loop with two Flow Elements

    - ARC Valves (Automatic ReCirculation Valve)

      - Operating Philosophy

      - ARC valves larger than 8” are so expensive and should not be used. Use flow (or pressure) control loop for large sizes.

      - ARC valves shall be used on clean services only.

      - Is NRV Required?

    - Restriction Orifice

      - It is recommended for pump with power consumption of 25 kW or less.

    - It is recommended to have dedicated minimum flow protection line for each pump in the following cases:

      - Where pumps cannot be operated in shut-off pressure even for a short period of time, for example high pressure pump such as Sundyne pump, BFW Pump Hydrocracker or RHU feed pump.

      - Low temperature service pumps as those pumps need to be operated in minimum flow at start-up after maintenance until their services are stable, and then discharge valves should be opened gradually.

      - Two or more pumps are operating in parallel and:

        - Pumps have high ΔP or

        - Pumps are engine/turbine driven

  - Note: Very high ΔP pumps may have dedicated minimum flow lines. Otherwise, dedicated “Maintenance Line“ equipped with RO for each pump may be considered so that each pump can operate at ~30% of its capacity while the discharge isolation valve is closed.

  - When to provide minimum flow protection line: The following criteria may be followed:

    - Differential pressure multistage pumps higher than or equal to 35 bar

    - Large pumps with driver power higher than 160 kW

    - For process reason (turndown), flowrate lower than or equal 30 per cent of max flowrate

    - Level control valve at pump discharge line (To be studied case by case).

## Metering Pumps

- PSV

  - According to DEP 31.38.01.11: “Positive displacement pumps SHALL be safeguarded against a blocked outlet with a pressure-relief device. This SHALL not be an integral part of the pump and shall be in accordance with DEP 31.29.12.30-Gen. The relief valve SHALL be installed in a bypass between the discharge pipe upstream of the block valve and the suction vessel.

- Pulsation Damper

  - Use pulsation damper at discharge to reduce the pulsation, if necessary.

  - Back Pressure Valve or RO can be used at discharge in combination to pulsation damper to have smoother flow rate

  - Pulsation damper can be used at suction line to reduce the acceleration head and consequently increase the NPSHa

- Material of the upstream drum and the suction piping shall be rust-proof (e.g., stainless steel or FRP/Plastic) to avoid rust entry to the positive displacement pump.

- Pressure Gauge/Transmitter/switch for double diaphragm pumps

## Steam Turbine

- Have a look on IPS-D-PI-112 and below sketch.

- Provide a pressure-relief valve between the turbine exhaust and the outlet isolation valve (Applicable to back-pressure turbines).

- Provide an isolating valve and a check valve on all turbines exhausting to a steam header.

- Provide an isolating valve on all turbine inlet lines (to be provided in addition to the turbine trip/throttle valve).

- A second isolating valve and spectacle blind may be provided in the inlet line at the header (up-stream of the FT).

- Provide a spectacle blind at the isolating valve in the exhaust line in all cases (a second valve is not to be provided at the header).

- Provide a warming bypass around the inlet isolating valve with one isolating valve and one throttling valve. This warm-up line shall be minimum 1” with typical size of 2”.

- Provide inlet and outlet PI on all turbines at the appropriate locations.

- Provide inlet and outlet TW or TI on all turbines in the appropriate location.

- Provide an inlet FT or other client requirement on all turbines in the location shown. Do not duplicate with a unit limits FQI when the turbine is the only user in the unit.

- Casing trap station is not required for manual start-up turbines.

- Indicate the existence of drains with valves and leak-off lines from turbines.

- Inlet separators are not required in the inlet line because a warming bypass is being included around the inlet isolating valve for start-up.

- Indicate requirements for an inlet line steam trap station (with drain) up-stream of the isolating valve by use of a note.

- When required, indicate remote shut-down.

- For automatic starting turbines, indicate the on/off start-up valve at this location and include a steam trap assembly for the valve casing drain.

- For compressors, show temperature indication on local panel(s) with remote (main) panel alarm on compressor discharge applications as high temperature alarm only.

- See also API RP 686; 2009 (Figure B.2)

Typical Arrangement for Condensing Turbine

## Compressors

- One purge-out line shall be considered at discharge line of centrifugal/reciprocating compressor (discharge of each stage). The by-pass line of discharge PSV may be used for this purpose.

Also, another purge-out line shall be considered at suction line of reciprocating compressor (suction of each stage). If a line connecting discharge to suction (with a normally closed valve) is available, the purge-out connection on suction may be eliminated.

- As for compressor (and some other packages), alarm and trip signal are indicated on the UCP/LCP. Depending on the company policies, the following signals may be repeated in control room (DCS work station):

  - Common alarm

  - Common shut-down

  - Common shut-down alarm

  - Stop/Running

  - Ready to start

It is expected that operator explores cause of the failure at the UCP and takes appropriate actions in accordance with the operating manual.

- Parallel operating reciprocating compressors should have common suction K.O. drum. While parallel operating centrifugal compressors should have dedicated suction K.O. drum.

- Friction Loss in strainer or flow meter of anti-surge loop to be considered.

- Friction loss in lube oil removal to be considered.

- To reduce the consequences of a fire in compressor area, remote activated isolation valves shall be installed in the suction and discharge of centrifugal compressors with a power higher than or equal to 150 kW and handling flammable or toxic gases. These valves:

  - Shall be fire resistant

  - Should be located so that they do not interfere with anti-surge facilities. These valves should be failed close (FC)

The alternative design is to use delay for these on-off valves (with electrical actuator (Fail Last)) to avoid interference with anti-surge system meaning that the valve shall start to close after 120 seconds (Typical, to be confirmed by compressor vendor) after compressor trip. In this case, the MOVs shall NOT have any handwheel or close hand switches and shall be activated only via ESD. Vendor should advise if

  - Shall be located so that they do not breach the requirements of API 521-Annex B, if a system (multiple equipment) is protected by a single PSV.

A remote operated emergency depressurizing system is recommended for centrifugal compressors.

- For a fast response of anti-surge system, the gas volume at discharge shall be minimized. Where it is not practical, a hot recycle may be required in addition to the cold recycle. The final decision should be made based on dynamic simulation.

- Be careful of condensation in anti-surge line of centrifugal compressors.

## Shell & Tube Exchangers

- Test Well and PI connection (Pressure/Temperature survey should be possible)

  - PI/TI connections on inlet/outlet nozzles:

    - TI connection (Test Well) should be considered for inlet/outlet nozzles≥4” (6” in some projects) if a TI is not considered on the respective line.

    - PI connection shall be considered for inlet/outlet nozzles≥2” unless the exchanger has its individual vent and drain connections on both sides.

  - Cooling water inlet nozzle does not require TI connection as the cooling water supply temperature is usually measured at battery limit. Since a TG is provided on CWR line from each individual water cooler, temperature connection on CWR nozzle is NOT required.

  - For stacked heat exchangers only one exchanger is equipped with TI/PI connection on interconnecting nozzles.

- Inlet/outlet location

  - Heating fluid usually enters at top and exits at bottom

  - Cooling fluid usually enters at bottom and exits at top

- Drain

  - Provide drain (and vent) connection for shell & tube exchangers with S type head:

Indication on P&ID

  - In case a heat exchanger is isolable, evaluate the need for utility connection, vent and drain to safe location if the fluid is volatile, flammable or lethal.

- Provide pressure-relief protection (generally in the outlet line) of liquid-filled services with heating media on the opposite side, when the stream can be blocked in by valves

- Should isolating valves are considered for water-coolers and the process fluid vapor pressure at the lowest actual CWS temperature is less than barometric pressure, exchanger shall be designed for full vacuum

- Generally, spectacle blind is not mandatory at cooling water side of water coolers.

- Isolating valves shall be provided on inlet and outlet lines where maintenance is to be performed on the exchanger with the unit operating.

- Provide PI on process streams entering and leaving exchangers or groups of exchangers as required (non-fouling services will generally not require the PI).  Provide a local PI in steam services between the control valve and the exchanger inlet.

## Water Coolers

- Bypass

Where a branch line for a single service rises from underground, block valves shall be provided in the risers just above the ground. The following arrangement will provide protection against freezing:

A by-pass shall be provided just under the block valves, from the supply back to the return for use in maintaining circulation in cold weather. This by-pass shall be DN 20 (¾") for lines DN 80 (3") and smaller, DN 25 (1") for lines DN 100 (4") to DN 200 (8") and DN 40 (1½") for lines larger than DN 200 (8"). Bypasses shall be covered with 25 mm of insulation. As per IPS-E-PI-240, type of the valve on this bypass line should be globe.

A drain shall be provided in the line at a minimum distance above the block valve, except that for 150 mm and larger size valves a drain shall be provided in the valve body above the seat.

25 mm of insulation shall be provided around the piping, from the ground up to and including the block valves in the water risers.

- Consider regulating valve at cooling water return line.

- Spectacle Blind/Spacer is not required for cooling water services.

- It is recommended to consider the water isolating valve type as follows:

  - Gate (supply) and globe (return) for water line sizes up to 10” (Inclusive)

  - Butterfly valves for size larger than 10”

- Shall we insulate water coolers?

- Cooling Water regulation:

  - Any regulation valve shall be installed on the CWR line

  - Avoid using automatic control of cooling water flow where possible. It is to avoid potential low cooling water flow which may result in severe fouling.

- Four-way back flush valves (Double L-port plug type) is required where cooling water quality is low (high TDS) and the cooling water line size is 2”~12”. For cooling water branches of smaller than 2”, the drain connection size at supply side should be same as cooling water branch size. Note that 4-way back flush valve shall NOT be used for surface condensers or where interrupt in the exchanger operation results in unit shut down.

- The drain sizes on water side may be as follows:

Cooling water Supply Line

  - CWS size < 4”:		Drain Size:	1” (*)

  - CWS Size 4~10”		Drain Size:	1 ½”

  - CWS Size >12”		Drain Size:	2”

(*) Where cooling water has high TDS (4-way back flush valves are used for water coolers), the drain on cooling water supply line shall be equal to the CWS line to exchanger for cooling water supply size up to 1 ½ “, inclusive.

Cooling Water Return Line: 	Drain Size:	¾”

## Air-coolers

- Accessories

  - Louver

  - Fan and Fan driver

  - Vent, Drain

  - Heating coil

- Instrumentation

  - Louver actuator, if any.

Refer to API 661 (ISO 13706), paragraph 7.2.3.11:

The louver position upon loss of control-air pressure shall be specified by the purchaser.

  - Variable pitch fans and fan pitch adjustment

Refer to API 661 (ISO 13706), paragraph 7.2.3.11:

For fans having a diameter larger than 1.5m, individual fan blades shall be manually adjustable for varying blade pitch. The use of automatic control for varying the blade pitch shall be specified by the purchaser.

Refer to API 661 (ISO 13706), paragraph 7.2.3.11-e:

The purchaser shall specify the direction of change of the fan pitch with loss of control-air pressure.

  - Fan speed control

  - Vibration switches (buzzer or alarm can be common for all fans)

- A common DCS alarm for failure of fans due to high vibration of one air-cooler is enough. It is recommended to stop every fan in case of high vibration of the fan via MCC.

- Number of bays, fans

- Size of nozzles

- Venting of non-condensable gases

- Indicate vents and drains with valves for bundle headers.

- Whenever steam coil is required (to avoid solidification or congealing), louvers shall be provided too.

A rough approximation for the steam requirement can be obtained for 9 m tube length by using 45 kg/hr of steam per meter of bundle width. Consider 2500 kg/h as the maximum figure. Note that every bundle should have its own coil, isolation valves, steam traps, etc.

- Invertor Control, Fan Pitch Control or Louver Control?

- Provide automatic fan speed control (or variable pitch control) for 50% of motors where precise temperature control at air-cooler outlet is necessary. However, the number of variable speed motors (or fans with automatic adjustable pitch) should be limited to eight (8).

In case air-cooler is followed by a trim cooler, locate the temperature element downstream of the trim-cooler. In the latter case, provide a TI with TAH alarm.

## Fired Heaters

- Fuel Cut-off: Pilot or Main burner?

- Flow monitoring on two phase flows

- The damper at upstream of induced fan is preferred to be equipped with HIC.

- Instrumentation: Refer to Exxon IP 15-1-1, API 556 and IPS-E-PR-810.

- Instruments for Flue Gas Monitoring System (as per standard)

- Temperature Instruments:

  - Inlet line

  - Outlets from each pass

  - Combined header from different passes of a heater

  - Header pass points from the convection to the radiant sections (Cross Over) needs TT on each pass.

  - On the stack, just ahead of the damper (TT)

  - On the air leaving the APH

  - Flue gas at Arc

  - Flue gas entering the APH

  - Combustion air to burners (downstream of by-pass air flow)

- Burner Management System (BMS) might be considered for Process Heaters to:

  - Prevent firing unless a satisfactory furnace purge has first been completed.

  - Monitor and control the correct component sequencing during start-up and shut-down of the equipment.

- Pressure Instruments

  - Discharge of FD and ID fans

  - Pressure drop of air preheater (flue gas side)

  - Suction pressure of the ID fan (PT to detect loss of ID fan)

  - Combustion air supply duct to the burners

  - Furnace fuel oil, fuel gas, and atomizing steam branch headers

  - A pressure transmitter shall be provided at top of the radiation section. This will part of the pressure control closed loop. Where air preheater is provided, a high high pressure switch at arc zone may be applied to cut-off the fuels.

  - Each furnace should use a single draft gage manifolded to the radiation section inlet (burner level), top of the radiation section (Arc or Bridge), below and top of the stack damper. In case there are more than one coil bank, the downstream pressure of each bank should be measured.

  - Downstream of individual valves installed at heater inlet, where applicable (pressure gauge or transmitter should be used).

Important Notes:

  - Long/large fireboxes may require several bridge wall temperature measurements along the length of the box. These measurements may be useful in assessing the uniformity of heat distribution along the length of the firebox.

  - Flue gas temperature measurement within the convection section may be useful when multiple services are located there. Flue gas thermocouples may be located between the different services to cross-check their heat absorbed duties.

  - Stack temperature measurement is useful both for monitoring the heater condition (trending of typical operation) and as a component used to monitor efficiency. The portion of the breeching where the thermocouple is installed may not be representative of the fully mixed flue gas in the stack. Some areas of breeching may be in low flow zones and not representative of fully mixed flue gas in the stack. If a common stack is used with several heaters, each heater should have a temperature measurement in the ducting to the common stack. The common stack should still have a temperature measurement point to monitor heater efficiency and environmental performance. The common stack temperature measurement will also be useful to detect afterburning.

  - One oxygen analyzer should be provided for each independent heater combustion zone. For large combustion zones one analyzer for every 10 m of firebox length is recommended due to non-uniformities in the firebox flue gas circulation and to facilitate balancing the burners. Oxygen measurements should be taken as near as possible to the point where combustion is completed, normally at the exit of the radiant section and before the transition to the convection section to avoid tramp air.

  - Thermowells on process side should be removable for pigging or decoking, where applicable.

  - When tube skin thermocouples are considered, the recommended minimum is two tube-skin thermocouples per pass.

- Weep hole for snuffing steam line to be considered.

- According to IPS-E-PR-340: “A flame arrestor trap of an approved type shall be fitted in the main foul or waste gas lines leading to a furnace, with a high temperature alarm actuator installed immediately downstream of the trap. Cleaning of the traps shall be provided.”

Note that the above requirement is for off-gases with low pressure on which no automatic isolation valves (to be closed in case of low low pressure of the off-gas) are provided.

## Drums

- Consider permanent piping for steam-out if steam-out nozzle is larger than 1”.

- Slope if required

- Review arrangement for draining of lines connected to dip pipe. Check if draining of line may result in draining of the whole drum inventory.

- Drum can be drained via the bottom line but consider separate drain line if the bottom pipe is not flush with the lowest point of vessel.

- If drum contains a baffle which divides the vessel in two sections, ensure both sides can be drained. Add separate drain line if necessary.

- Pressure instruments and PSVs are preferred to be installed on vapor outlet line (not on the vessel).

- Shall we consider both LLL and LLLL?

- Measuring hydrocarbon level in skimming box: Take care of the legs for LG (or LT)

- Cladded Vessels:

In case the vessel has cladding up to a certain level, the steam out connection and manways should be provided above the cladding. Manways may be installed on top of the horizontal vessels.

The gate valve on steam-out line which is connected to the vessel (and the isolating valves of level instruments may need to be of superior trim material such as Monel. It should be checked by material selection specialist.

- Ensure the catalyst/adsorbent withdrawal nozzle are provided at the right location.

- Have a look on Shell DEP 31.22.05.11; Article 4: PIPING REQUIREMENTS (Just for information).

  - Sprinkler

- Show sprinklers, distributer or injection nozzles as they may have a significant pressure drop.

## Storage Tanks

- Low low liquid level (Refer to JGC STD-09-025)

- Low liquid level

  - Mind the Concept

  - Mixers shall trip

- Blanketing Sensing Nozzle

- Limit the open vent nozzle size to 12” (Maximum 16”). Increase the number of the open vents, if necessary.

- Do not consider inlet/outlet lines for PVRVs unless it is necessary (for example on lethal or very toxic services).

- Drains

  - Floating Roof Emergency Drain

  - Tank Drain

  - Drain pit

## Cooling Tower

- Is Minimum flow protection required?

- Where to take blowdown stream from?

- Basin for sand filter back-wash

- Is hot by-pass line acceptable?

# Equipment Trim Number

Trim number refers to specification of piping for instruments and vent/drain of equipment.

Note that minimum design pressure specified for trim number of atmospheric storage tanks should be 3.5 barg.

# Equipment Elevation

  - Water Coolers

- Closed Loop

- Once-through

- Fiberglass piping

Be careful of cooling tower height versus water coolers in plant.

Attention: If bypass connection for cooling tower is considered, even water coolers lower than water distribution flume will be vacuumed in case cooling water pumps trip and the bypass valve is open.

Try to avoid overflow for closed circuits and cooling water networks in case of pump stop. This can be done by changing the buffer drum elevation or increasing the holdup volume between NLL and HLL.

If sea water is directly discharged from a water cooler installed at a high point to an open ditch, the cooling water return shall be routed to a funnel immediately downstream of the water outlet valve which is installed at the same level of the water cooler. It is to avoid vacuum formation in the cooler in case of cooling water failure.

  - Two Phase Flow

- To specify whether a pipe contain 2 phase flow or not

- Free Draining

- Size modification

- Hot Steam Condensate (2 phase)

  - Catalyst/Adsorbent withdrawal nozzle

  - Saturated liquid from vessel to flow element or control valve (Minimum 3000 mm above HPP). Please note that:

- It is mandatory for flow measurement

- It is preferred for control valves to avoid cavitation. Therefore, for flashing services, this requirement will be ignored.

  - Pump NPSH

- Liquid containing gases (e.g., deaerator, bottom outlet of steam stripper):

Do not forget to consider enough margin (or correction factor). For instance: Refer to Technip or Shell DEP.

  - Intake filter for blowers/compressors sucking air from atmosphere (Minimum Recommended: 4 meter)

  - What is the difference between “Free Draining” and “No Pocket”?

- Free draining results in No Liquid Pocket.

- Slope of free draining lines may or may not be specified on P&IDs. Attention! Line cannot be installed absolutely horizontally.

  - Free draining

- Chloride or sour gas which may condense

- Fired Heater Transfer Line

- Remote sensing line for pilot type PSVs

- Flare lines

- Saturated vapors including vapor outlet line for blanketing system

- Balance/equalizing line either for steam traps or condensate pot

- Two phase flows: It is much easier to size a line for two phase flow if the line is free draining provided that it is practical and economical,

- Suction line of compressor (From upstream KOD to compressor should have no liquid pocket)

Note: Some items e.g., rotameters shall be installed upward. Hence, it is not possible to keep free draining from upstream to downstream.

  - Check for upward section for lines handling saturated liquid

  - Liquid draw-off from distillation towers

  - LP condensate flash drum and condenser:

It is very important to ensure that the flow direction of flashed steam to the condenser. Note: it is usually economical to condense the flashed steam instead of sending the steam to atmosphere. This is true for even small steam flow rates.

The above scheme is NOT ACCEPTED because:

- Severe Pump cavitation may be observed even with ample NPSHa. This is due to entrainment of steam bubbles in pump suction. Line from condenser shall be dip pipe and located as far as possible from pump suction nozzle.

- Condenser will not work properly.

Recommended Schemes are as follows.

Scheme 1:

Scheme 2:

The Dip Pipe shall end below the lowest liquid level.

Note that in any of the above schemes:

- The aircooler elevation should be finalized based on the results of hydraulic calculation from aircooler outlet to the condensate flash drum. It means the return line from aircooler to the flash drum should be large enough to handle the condensate flow by the available height difference

- The line from the flash drum to the aircooler and the line from the aircooler downstream to the atmosphere should handle overpressure scenarios.

| *1 | Accumulated inert gas vent line |
| --- | --- |
| *2 | The elevation of the condenser should be sufficiently lower than that of the overhead receiver to maintain the required pressure drop across the control valve on the hot bypass line. In case of the hot bypass control, the condenser is generally installed at grade. |

# Isolation Philosophy and Removable Spool Pieces

  - Isolation Purpose

- Isolation devices can be installed for the following main functions:

- To enable an entire system (made of equipment items, associated pipework and instruments…) to be taken out of service safely while other systems may continue to operate normally. This includes:

- Separation of process sections from other live process equipment,

- Separation between process and draining facilities,

- Separation between process and flaring/ venting facilities,

- Separation between process and utility systems.

- For maintenance or inspection purpose:

- To provide physical barriers before personnel entry is permitted,

- To provide means to safely reinstate the equipment or system following a successful maintenance or inspection/ testing operation.

  - Isolation category

- Isolation at battery limit

- Isolation for maintenance during plant shut-down.

- Isolation for on-stream maintenance.

## Isolation at battery Limit:

Unless otherwise specified in project specifications, the following arrangements may be used.

Notes:

      - Arrangement I is for single block valve. Using dingle block or double block arrangement should be as per project specification.

      - Arrangement II & III is recommended for double block isolation. Although IPS recommends arrangement II for utility streams and arrangement III for process streams, arrangement II should be used for lines entering the battery limit where the interconnecting line will be still pressurized during unit shut-down.

      - Arrangement IV should be used for:

        - LPS (OP<6 barg) if the line size is equal or larger than 10”

        - MPS and HPS if the main line size is 6” or larger.

External by-pass is not required if main isolating valve is provided with an internal by-pass.

  - Check valves are shown typically only. Provide check valves only where necessary.

## Isolation for on-stream maintenance

Positive isolation shall be provided on all lines as close as practical to the equipment.

## Isolation for maintenance during plant shut-down.

Shutdown means that the plant, unit or system has been isolated, and that it is at atmospheric pressure, drained, steamed out or purged and free of hydrocarbons or chemicals other than catalysts in reactor vessels.

For the complete unit shutdown, all the feeds, products and utility lines will need to be isolated at the unit battery limits.

Some equipment are essential part of the unit meaning that they CANNOT be isolated while other sections are in operation. Personnel entry to such equipment happens only during plant shut-down. If such equipment need positive isolation for personnel entry, vapor blind or insertion plates may be used.

The following table is a sample for isolation of equipment. Note that in case of using a removable spool piece, neither SB nor RS is required.

LIMITS OF USE OF SPECTACLE BLINDS/BLINDS AND SPACERS, REMOVABLE SPOOLS (TYPICAL)

| NPS |  | ANSI/API FLANGE RATING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (Inch) | 150 | 300 | 600 | 900 | 1500 | 2500 |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  | SPECTACLE BLINDS |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 12 |  |  |  | SPADE AND SPACER / REMOVABLE SPOOL |  |  |  |
| 14 |  |  |  |  |  |  |  |
| OVER |  |  |  |  |  |  |  |

(Selections in above table are based on using spade and spacer when weight of spectacle blind exceeds 50 kg).

  - Either Spectacle Blind, Spade or Removable spool Piece shall be used for isolating an equipment for personnel entry (Positive Isolation).

  - In case of using removable spool piece, there is no need to consider spectacle blind or spacer/spade.

Use of spectacle blind or spade/spacer should be avoided in RTJ. Removable spool piece (at a right angle) should be used instead.

  - As a general rule, if frequent access is required, double block and bleed is preferred to spectacle blind unless the service is very toxic or the operating pressure is too high or personnel entry is required.

  - Insertion plates may be used for positive isolation as vapor blind (Not pressure blind).

  - Air-coolers

- Bundle isolation is not required unless for the following services:

  - Fouling Services

  - Plugging Services

  - Corrosive Services

  - Erosive Services

For the above-mentioned services, purging connection for each bundle is recommended.

  - Exchangers

- As per isolation table.

- It is not necessary to provide isolating ring spacers on column reboilers if adequate isolation is provided at column.

- Removable spool piece may be required at tank heater tube side or exchangers with removable bundle (tube side) to facilitate removal of bundle. Refer to project specifications.

  - Pumps

- API RP 686 (2009); §2.3: Isolation using two closed block valves and a vent valve open to atmosphere located between the block valves is known as a “double block and bleed” arrangement. This is an acceptable alternative to spectacle blinds for machinery isolation, provided the process fluid is not toxic, corrosive, or flammable. Removal of a breakout spool and installation of a blind flange is also an acceptable alternative to the use of spectacle blinds for machinery isolation.

- Generally, spectacle blind is not necessary for pumps unless in lethal or very toxic services.

  - PSVs:

- While spectacle blind is not necessary for PSVs, it may be the cause of the unsafe situation, if the SB is left closed in operation. Therefore, SB or spade shall NOT be provided on inlet/outlet line of PSVs. The PSV isolation valves and the bleeders at upstream and downstream of the PSV are sufficient for the isolation purpose.

Where PSV has a bypass and it discharges to a closed system (e.g., flare), provide SB upstream of the bypass valve. If there are two valves in series on the PSV bypass line, the SB may be installed between the bypass valves. The SB on the bypass line shall be normally open. It will be closed only for maintenance.

- The alternative to the above is to provide a removable spool piece on the inlet line to PSV. This method is applicable if there is a dedicated line for PSV and its bypass from the vessel/filter. In this case, the spool is installed immediately on the equipment.

  - When to have Spectacle Blinds at open position (Slop, Pump Vent, Equipment Drain …)?

  - Here below are some examples for SB application for process purpose:

- By-pass of mole-sieves

- On any air connection to HC services (for example air connection for catalyst regeneration regeneration).

  - Vapor Blind or Pressure Blind? (See IPS-D-PI-119 & IPS-D-PI-120)

  - Removable spool piece may be required at:

- Inlet Steam to turbine (for flushing)

- Line which is connected to the top nozzle of filter for pulling out of inside mesh. This also the case for line(s) connected to drums with top body flange where the body flange is as manhole.

- Piping which is connected to mixer driver for mixer located inside equipment

- Piping connected to the vessel nozzle with removable dip pipe

- Analyzer elements to remove of probe insert (where analyzer need to be retractable). Consult instrument specialists.

- Suction and discharge of compressors

- Injection points

(Removable spool pieces on lines with RTJ flanges shall be installed at elbows).

  - Swing elbow may be required:

- At PSV inlet line where PSV discharges to different locations depending on operation mode.

- To avoid complex piping arrangement where change in fluid destination happens rarely.

  - Isolation at battery Limit: Unless otherwise specified in project specifications, the following arrangements may be used.

Notes:

      - Arrangement I is for single block valve. Using dingle block or double block arrangement should be as per project specification.

      - Arrangement II & III is recommended for double block isolation. Although IPS recommends arrangement II for utility streams and arrangement III for process streams, arrangement II should be used for lines entering the battery limit where the interconnecting line will be still pressurized during unit shut-down.

      - Arrangement IV should be used for:

        - LPS (OP<6 barg) if the line size is equal or larger than 10”

        - MPS and HPS if the main line size is 6” or larger.

External by-pass is not required if main isolating valve is provided with an internal by-pass.

      - Check valves are shown typically only. Provide check valves only where necessary.

# Emergency Isolation

  - Valves for Liquid Containing Vessels

- Requirement: As per project specification e.g., IPS-E-PR-470 or NIOEC SP-0050

- The valve shall be located as close as possible to vessel.

- Valve should be Fire resistant.

- Valve shall also have a local hand switch at safe location distance.

  - Centrifugal Compressors

  - Fired Heaters: As per project specification e.g., IPS-E-PR-470

- Consider three on-off valves. Two of them on the fuel gas to burners (TSO if required by project specifications) and the third one to discharge the gas between aforementioned valve to atmosphere (Safe location)

- The same arrangement should be considered for fuel gas to pilots

# Warm-up/Start-up

  - Centrifugal Pumps: A bypass globe valve (1”) shall be installed in the following cases (See API RP 686; §2009; 2.11):

- Operating temperature > 150°C

- If discharge/suction pipe operating temperature is below -10 °C.

- Clogging, congealing or freezing possible

The warm-up/cool-down valve may be installed either around check valve or around check valve and the discharge isolation valve both.

In some services, the by-pass valve, if necessary, shall be strictly around check valve, only. It means that the discharge isolation valve shall not be bypassed. This is to prevent fluid leakage in case the pump is to be isolated for maintenance:

  - For pumps handling highly volatile liquids at pumping temperatures, e.g., LPG service. In this case, the bypass valve shall be only around check valve i.e., the downstream isolating valve shall not be by-passed.

  - where the fluid is lethal, toxic or above its auto-ignition temperature.

For other services, the arrangement of warm-up line (whether block valve is by-passed or not) may be as per project specification or client preference.

Sometimes, a check valve with a hole of 3 mm to 5 mm (0.12 in. to 0.2 in.) diameter in the closing member of the check valve is be used instead (Not recommended unless for vertical discharge pumps).

- Barrel Type Pumps (Warm-up and Minimum Flow Protection)

  - HPS/MPS @ BL (Applicable when line size is ≥ 6”; Use a 2” line for warm up equipped with one globe and one gate valve in series. The gate valve should be installed at battery limit side.)

It is preferred to have a warm-up bypass on LPS isolation at battery limit when the LPS header size is ≥ 10”.

  - HPS/MPS @ Steam heaters

  - Steam to Steam Turbine

- Use a 2” bypass for the main manual isolating valve equipped with a gate valve and a globe valve in series.

- Provide dry-out connection (Recommended Size: 2”): It is preferred to connect it to a silencer.

  - Start-up bypass line for a once-through type reboiler using sieve or valve type trays and NOT using an accumulator tray

The startup bypass line size is determined based on “50% of the circulation flow rate”. However, this should be less than the bottom product line size.

For

Start-up

Bottom Product

  - Centrifugal Compressor

Centrifugal compressors shall be pressurized gradually to avoid seal failure.

  - Pressurizing Line for Towers (Start-up)

  - Beds/Reactors filling line (Liquid and Gas Services)

Gaseous Beds

Notes:

- The flow direction in liquid phase beds is usually upwards.

- Sample connections shall be placed so that the sample is always fresh.

# Liquid Seals for Vapor Condensation or Level Control

  - Seal Height

- Vapor condensation: Height of seal should be equal to exchanger pressure drop

- Level Control: Height of seal is almost equal to the normal liquid level.

  - Siphon-breaker for liquid seals

  - Note that the distance/elevation of pipe is measured from its centerline. Should we mean the pipe top/bottom, it should be noted on P&ID clearly.

# Details of Injection and Sampling Point (for analyzer)

  - Injection Point

  - Sample point of Analyzer

# Full Symmetrical Piping

  - Piping around air-coolers with two phase flows at inlet/outlet shall be fully symmetrical

  - Piping around fired heaters shall be fully symmetrical unless individual control valve is considered on each pass.

  - Piping around exchangers with X, J or H type shell should be full symmetrical. Deviation may be acceptable for single phase inlet/outlet after hydraulic study.

  - Parallel exchangers (Including thermosiphons)

  - Inlet to towers

  - Inlet to horizontal (Flare) KO drums with two inlet lines (and similarly outlet from horizontal KO drums with two outlet lines)

# Reducers

  - Type of reducer (Concentric/Eccentric): It is recommended to specify the reducer type if there is any requirement from process point of view.

- Flow to Air-coolers (installed horizontally)

  - Inlet:	Bottom Flat shall be used. This is to facilitate draining of upstream line via air-cooler.

  - Outlet:	Top flat is preferred.

- Pump Suction (Side/End suction or Top suction)

  - The reducer for side or end suction pump shall be top flat

- Slurry Service: use bottom flat reducers

- When horizontal reducers are installed in the inlet piping to compressors or blowers, they shall be eccentric with the flat side on the bottom of the pipe to prevent the accumulation of any liquids (API RP 686).

- Steam Lines (especially steam to turbine): Reducers shall be bottom flat.

- Services in which fluid entrainment should be avoided for safe maintenance (e.g., for lines containing lethal or pyrophoric liquids): Use bottom flat reducers.

- If reducer/expander is required for TW installation, they should be bottom-flat type.

- Sometimes, using an eccentric reducer is a piping requirement. For instance, reducers of lines on pipe rack are most often bottom-flat type.  This is to maintain a constant bottom of pipe.

  - Number of reducers

  - Reduced Bore Flange (RBF)

  - Location of reducers for parallel equipment

  - Size of equipment nozzle shall not be less than the respective line size for:

- Shell & Tube Heat Exchangers

- Vessels

- Tower

- Reactors

Recheck, if the equipment nozzle size is smaller than the corresponding line size.

This is a process requirement. Besides, it makes piping design easier.

  - Reducers are usually not required on vessel outlet line unless where:

- PSV is installed on the vapor outlet line and the pressure drop at PSV inlet lines dictates so.

# Check Valves or Non-Return Valves (NRV)

  - Check Valve to be considered for: (See API RP 686; § 2.9.1)

- Centrifugal Compressors

For centrifugal compressors, the NRV shall be installed downstream of the anti-surge line branch point. (See API RP 686; § 3.2.3.8). The intent of this requirement is to prevent over pressuring the compressor suction line with discharge gas when the compressor is shutdown. Two check valves in series are requirement.

- Centrifugal Pumps

- Vent Line from Pump Seal Pots

- Reciprocating compressors (Spare compressor will be started off-loaded).

- Pumps/compressors sending fluid to two or more destinations (Including Pump Minimum Flow Line or Compressor Kick-back line)

- Discharge of metering pump, Downstream of PSV connection

- On BFW line to desuperheaters

- Instrument air to reservoirs

- Flare Lines:

  - Pressure Control Valve when normal operating pressure is less than max flare back pressure

  - Drain to flares

  - Vent connection of level instrument. NOTE: In this case, one check valve will be considered for the whole level instruments on a vessel/tower.

  - Sample connections with sample return line

- Fuel Oil Return lines

- Steam/Nitrogen Connections to pipes/equipment in other services.

- Steam/Nitrogen to process lines, vessels or towers (e.g., steam strippers)

- On all turbines exhausting to a steam header

- Steam trap discharge lines when trap discharges to condensate header. This is recommended only when the respective exchanger operates intermittently and its operation starts automatically.

  - Number of Check Valves

- There should be two dissimilar check valves on compressor discharge lines if gas combines with liquid

- Pump Discharge (High discharge pressures): As per project specification

  - Non-slam check valves to be considered at all compressors discharge line (Mushroom type is preferred).

  - As per API RP 686 (2009); section 3.1.1.2: “Non-slam check valves shall be used in the discharge lines of centrifugal pumps in large systems. Acceptable non-slam check valves include wafer-style center-guided spring-loaded split-disc check valves or tilting-disc check valves.

NOTE: Large systems are typically those used to transfer water or other fluids in large volumes and/or long distances. Non-slam check valves should be considered for pumps with greater than 185 kilowatt (250 horsepower) nominal driver rating or NPS 12 or greater piping.”

  - NOTE Conventional swing check valves are typically acceptable for use in rotary screw compressor applications because pulsating flow variations are generally small.

Vent/Drain for Level Instruments (High Vapor Pressure Liquid)

The above configuration is suitable for low vapor pressure services. For high vapor pressure services, in which all fluid will be vaporized after depressurizing to flare, the drain connection is not required. However, utility connection at low point is still required.

# Line Finishing

  - For tracing, see section 15

  - Pipe Finishing is mostly:

- Painting:

  - Galvanized pipes are not painted.

  - Stainless steel pipes are not usually painted. However, in location with high humidity and chloride in atmosphere, it may be beneficial to consider painting for stainless steel equipment and piping.

- Insulation: Mainly Heat Conservation and Personnel Protection

  - When Personnel Protection is specified as finishing, pipe is insulated till reachable height only. Steam hot condensate line to flash drum may be in this category.

  - When Heat Conservation is specified as finishing, the whole pipe is insulated except flanges, valves, strainers, and all other piping elements which need access.

  - Continuity of tracing/insulation should be clarified on P&IDs.

  - Depending on project specification, insulation thickness for personnel protection and heat conservation may be different or the same.

  - Do not forget to insulate (Heat conservation) the hot bypass line on the tower overhead from the control valve up to the receiver.

# Heat Tracing

  - Steam or Electrical

Electric tracing should be used in preference to steam tracing under the following conditions:

- Where the temperature must be accurately controlled or limited.

  - Water lines to safety showers and eyewash fountains because of safety hazards if line is overheated.

  - HF, H2SO4 and NaOH lines in certain concentrations because of corrosion or stress-corrosion cracking at elevated temperatures. (Steam tracing with spacer may also be used).

  - Fuel oil lines where high steam temperatures would cause coke formation and fouling of the line.

  - Adsorption package inlet Line (e.g., PSA). Note that if adsorbent is designed for gas phase, it will have poor performance even if with small amounts of liquid contained in the inlet stream. Therefore, saturated gas sent to PSA/Adsorption shall be traced (Preferably electrical tracing).

  - Normally No Flow Lines (NNF): Use electrical tracing if high temperature may cause upset/damage to downstream equipment or sometimes degradation of the component. The latter is applicable to only very few cases. Note that:

    - Normally, there is no temperature control when using steam tracing

    - In case of no flow, the fluid temperature will increase.

- When lined pipes are used to avoid corrosion and/or abrasion. Most rubber and polymer linings should not exceed 93°C.

- Be careful of design temperature while choosing tracing type. Cables which are suitable for high temperature may be much more expensive.

  - When an equipment needs tracing, it should be specified on the P&ID. Besides, the type (Steam/Electrical), and maintain temperature should be mentioned. For steam tracing, it should be noted if any steam other than LPS is required.

  - Some lines such as BFW may not need tracing but pressure instruments or DPT (e.g., for DP type flowmeters) lines/tubes may need tracing to avoid freezing.

  - Heat Conservation or Winterizing

- Winterizing: Need for tracing during cold ambient temperature (Refer to the KBR winterizing guideline)

Winterization can be provided in several different ways. Generally, the least expensive method that is practical for all plant operating modes should be chosen. The following list of winterization methods is given in order of increasing cost.

  - Vary liquid composition to avoid freezing

  - Provide for draining of pipe and equipment when not in service. Draining includes opening a low point valve or designing the line to be free draining. Draining must be consistent with plant operating procedures and must be reasonable to accomplish in case of an unplanned shutdown.

  - Blow liquid out of the line by gas pressure. Blowing must be consistent with plant operating procedures and must be reasonable to accomplish in case of an unplanned shutdown.

  - Install bypass lines around equipment that may be isolated to maintain circulating flow in the pipe (such as water lines around exchangers).

  - Continuous circulation of fluid during temporary interruptions of the process. Continuous circulation must be consistent with plant operating procedures.

  - Insulate lines to preserve the available heat. Many lines that have available heat are already insulated for heat conservation. The heat conservation insulation may be sufficient for winterization.

  - Provide heat tracing with the appropriate insulation.

  - Provide heat jacketing.

  - Bury lines underground below the freeze line. This option is rarely used except for severe climates. Portions of cooling water and firewater lines are sometimes buried for other reasons and consequently are also protected from freezing.

Different portions of the same line or lines in the same service do not always need the same winterization technique. Above ground portions of mostly buried systems may require draining or heat tracing. A service constantly operating at elevated temperature may be in jeopardy during a planned or unplanned shutdown. Because a large line in this service takes longer to freeze, there may be enough time to drain the line for a prolonged shutdown. However, a small line in the same service may be heat traced because it freezes more quickly and draining it for all shutdowns would be considered unreasonable. The use of draining as a winterization technique needs to consider the line and equipment use. Draining would not be the proper winterization technique for a system in intermittent service, but that is needed immediately when required.

Drain lines usually do not need winterizing if the drain valve is installed closed to the main line (less than 5D).

- Heat Conservation: Need for all-time tracing:

  - Compressor Suction Lines (From Compressor KO drum to Compressor).

  - Fuel Gas Lines

  - Fuel Oil Lines (including flowmeters and pressure instruments)

  - Acid Flare Lines

Note: Tracing of large lines (say NPS≥60 may not be practical)

  - The following items shall not be traced:

- Temperature instruments

- Capillary tube of pressure instruments

  - Flow Instruments

- Differential Pressure Types

- Rotameters

Rotameters are winterized in accordance with the requirements of the process line in which they are installed. They are housed when they are recording and when they are outdoors.

Note that electrical tracers have to withstand the design temperature of the line/equipment even if operating/maintaining temperature is not high. This should be considered when tracing line/equipment with design temperature of more than 120°C or when high temperature can be experienced even for short periods.

  - Control Valves

Control valves are not traced with the associated process piping, except those valves are steam traced on gas or vapor services with high pressure drops, where hydrates may be formed, or where freezing or congealing may occur.

When control valves used as direct connected regulators require winterizing, the pressure control line and valve diaphragm chamber containing the process fluid shall be heat traced and insulated. When the diaphragm chamber is sealed, the pressure control line shall be heat traced and insulated from the point of seal to the process line connection.

In light hydrocarbon-vapor services, where hydrate formation or frosting due to low temperature is likely, only the control valves body shall be heat-traced.

Steam atomizing control valves with diaphragm and in contact with heavy process fluid shall be protected with seal pot filled with glycerin.

  - Analyzers

- Sample system for analyzers which require a liquid stream shall be protected in the same manner as the pipe line from which the sample is obtained, using caution to ensure the sample is not damaged by overheating or vaporizing.

- Gas samples to analyzers which contain condensable shall be provided with heat tracing to prevent condensation.

- Heated housing shall be provided for analyzer and with temperature control and sample conditioning systems. Each analyzer installation shall be investigated for winterizing requirements.

  - Water Seals

- Water seals should be steam traced. Special care must be taken for turbine auxiliaries.

  - Pumps

Winterizing is required for services containing water and other fluids with pour points above minus 18°C but less than 10°C. Pumps are not to be heat traced:

- Hydrocarbon pour point 0-10°C

Heat trace dead legs. Insulate but do not heat trace the lines to and from the pumps. Use backflow circulation to keep non-operating pump lines above pour point. Drain pumps when taken out of service during cold weather.

- Aqueous Fluids

Trace and insulate all lines to maintain 24°C if required by climatic or process conditions Use back-flow circulation through non-operating pumps. Drain pumps when taken out of service during cold weather.

  - Miscellaneous Items

Steam tracing shall not be used for potable water service.

# Strainers and Filters

  - Strainers

- Type (T, Y, Basket, Conical)

- According to DEP 31.38.01.11 (PIPING - GENERAL REQUIREMENTS), Y-type strainers shall be used for permanent installation in vertical pump suction pipes.

- Where to use:

    - Chemical filling (e.g., Ammonia to Refrigeration Package)

    - Upstream of liquid spray device (e.g., Injection Quill) and Restriction orifices.

    - Fuel Oil Lines to main burners

    - Fuel Gas Lines to pilot burners

    - Steam to turbines

    - Steam to ejectors (Preferred)

    - BFW to desuperheater (Preferred)

    - Centrifugal pump suction (Not necessary for the main cooling water pumps)

    - Reciprocating pump suction

    - Compressor Suction

    - Upstream of turbine flow meters (recommended in API 551, Sec 2.3.4.2.4)

    - Upstream of cage guided control valves or perforated-plug valves (High DP valves). If the service is continuous, piping arrangement shall be designed to enable operator to clean the strainer while the line is in service (e.g., the strainer shall be installed upstream of the control valve not the by-pass line).

    - Inlet of Steam Traps (Provide external strainer for ball float and inverted bucket even if equipped with as-built strainer. External strainer is not required for other trap types if equipped with as-built strainer.)

    - Inlet to Plate Heat Exchangers (Use basket filter for seawater side)

(See Exxon DP09L)

    - Upstream of small needle valves (e.g., in sampling packages)

    - Inlet of liquid distributor for packing and inlet of spray nozzle

    - Inlet to turbine flowmeter (Preferred)

    - Nitrogen gas for tank blanketing at upstream of PCV (Preferred)

- Cone type Installation Direction: The point of the conical strainer shall face upstream in the piping (Refer to API RP 686 Section 2.8.2.2)

- Drains of strainers to be shown (typically for ≥6”)

  - Filters

- Provide filter on feed lines to units especially if the feed is stored in floating roof tanks.

# Connection to/from Top of Line

  - Analyzers

  - Steam

  - Two phase steam Condensate

  - Instrument Air

  - Flare

  - Fuel Gas (even individual branch of each burner)

  - Suction line of compressors

  - By-pass line of filters

  - Hot by-pass line for tower pressure control

  - Potable Water (Preferred)

  - Sample connections (should be from top or side)

It is also preferred to connect the pipes at 45° for below services:

  - Flare tail pipe to header (for pipes equal or greater than 4”) [Piping issue]

  - Any drainage system including closed drain system

  - Two phase flows e.g., blow down streams from steam generation system (not mandatory)

# Minimum Required Distances & Distances to be minimized

Minimum Required Distances

  - Draining / venting of liquid which their temperature may fall below zero. (Only for operational drain and vent).

  - Desuperheater to the temperature element and straight run after desuperheaters: (If data from desuperheater vendor is not available, consider 15D downstream of the desuperheater and 5D upstream of the desuperheater).

  - Lines handling liquid draw-off from towers should have a vertical downward run. This is to compensate the pressure drop across the nozzle to have liquid phase fluid especially when the fluid is sent to pump, valve, flowmeter or exchanger. The pressure drop across the nozzle can be calculated; however, the typical downward run is 10 D or 3 m, whichever greater.

  - Downstream run of inline mixers, where degree of mixing is a concern, should be advised by the mixer vendor. Typically, it should be minimum 10D or 2 m whichever is greater.  When water injection into the main line is considered to prevent crystallization inside the air-cooler, at least 10 m straight run is required from the injection line to the air-cooler.

  - Downstream of angle valves with flashing (e.g., steam drum blow down): consider 10D straight length as minimum.

  - Manual Isolating valve on fuel line of fired heaters should be installed in safe location distance (minimum 15 m away from fired heater)

  - Minimum height for vessel which its saturated liquid is measured by a flowmeter is 1.7 m. In this case the allowable pressure drop of the flowmeter should be reduced (e.g., maximum 0.1 bar for orifice)

  - Minimum distance from a fired heater to the isolating valve installed on snuffing steam should be located at least 15m away from the heater (FW STD 506). This is also the case for emergency push buttons which shut an emergency UV or fired heater.

  - Distance from Equipment in Molecular Sieves or Hot/Cold Connections (Minimum 2000 mm)

  - Temperature instrument downstream of mixing of two fluids at different temperatures (such as by-pass line of heat exchangers): 10D

  - Minimum Upward piping to fractionation side-strippers

  - When the minimum flow bypass line is routed into the pump suction line, this re-entry to the suction line shall be at least five pipe diameters upstream of the pump suction nozzle (API RP 686).

  - Some engineering companies use for 10D or minimum 500mm downstream of control valves in flashing services. Note that the isolation valve (gate or ball) can be installed within the distance.

  - Barometric leg for vacuum system and intermittent steam heaters which should automatically start to operate

  - Compressor Discharge line connecting to a liquid (or two phase) line (Consider two dissimilar check valves)

  - Minimum required length for full symmetrical piping:

Minimum L:		4D

L

L

Minimum L:		No Concern

h (from high high liquid level to bottom of condenser), shall provide enough static head to overcome the condenser pressure drop, plus the control valve pressure drop, plus the overhead line pressure drop.

Distances to be Minimized

  - The distance between centrifugal compressor discharge nozzle and the anti-surge control valve (See API RP 686; § 3.2.3.5). The discharge line shall be designed such that the volume of gas in the line between the compressor flange and the anti-surge valve and the discharge check valve does not exceed the compressor manufacturer’s design limit.

  - Distance between centrifugal compressor and NRV installed at its discharge

  - The distance between sample point and analyzer/sampling device shall be minimum.

  - Flare header/sub-header Purging

  - Purging of line when powder may accumulate or polymer formation is likely.

  - Flame arresters (from the end side which fire is possible to happen): For example, if a flame arrestor is installed on fuel gas to a fired heater this should be close to fired heater. According to TOTAL (GS-SAF-262): “Flame arresters shall be located at the vent tip or less than 3 meters away from the tip and only if there is no bend or piping obstruction between the tip and the flame arrester… Flame arrester shall be accessible for maintenance without scaffolding and shall be fitted with bug screens. In order to facilitate the set-up of a maintenance program, flame arresters shall be itemized on the P&IDs.”

  - Valves in steam branches which are normally closed e.g., upstream block valve of steam-out lines

  - Distance between NRV/XV and connection point to a hazardous fluid

  - The distance between control valve and downstream PSV (when design pressure is reduced)

  - Length of suction line of self-priming pumps (Generally, suction line of pump should be minimized but suction lines of self-priming pumps need special attention.)

  - When rupture disk is installed at upstream of the PSV to avoid clogging at PSV inlet line, the rupture disk should be installed at minimum distance from the equipment to be protected.

  - When different fluids will be handled in one line, the isolation valves should be installed at minimum distance from the main line.

  - The temperature transmitter on the tower overhead line should be installed as close as possible to the tower.

  - When flushing oil is required (e.g., in VDU), the flushing oil connections should be installed so that the heavy hydrocarbon is flushed as much as possible. The valve on flush oil line shall be installed as close as possible to the process line.

  - Distance between pressure measurement point on fuel gas line to burners:

API 556; §3.2.2.4: The piping pressure drop from the pressure measurement point to the burner should be designed for no more than 1/2 psi at maximum heat release for all anticipated fuel gas compositions.

  - Pipe length between temperature transmitter and the downstream equipment (e.g., reactor, tower) where the temperature at the equipment inlet. Ensure the time lag is acceptable. If it is not practical to follow the above recommendation, provide additional transmitter (open loop) at minimum distance to the downstream equipment. Check the lag time with a control engineer, if in doubt. This requirement may be ignored if the control loop is used only during start-up.

  - For steam-water heater, the isolation valve of steam should be installed as close as possible to the mixer (ejector).

A sample sketch of ejector/mixer is shown below.

  - Distance from inlet isolation valve to the turbine inlet to be minimized

  - Location of control valve on feed line to tower

The control valve on feed line in the following service should be located at the minimum distance from the feed nozzle to prevent corrosion/erosion due to the vaporization of H2S.

# Utility Distribution Diagrams (UDD/UHD)

  - Fuel

- Refinery Fuel Gas

- Fuel Oil:

  - Check if there is any diesel engine (e.g., diesel generator, dual fuel gas turbine or diesel driven pump) which needs fuel oil/gasoil

  - Check if a middle distillate (e.g., gasoil) is required for flushing lines/items handling heavy hydrocarbons.

  - Cooling Water

- Watch the pressure profile in the plant.

  - Flare header

- It should be cleared if a flare KO drum is required for the process unit.

- Valve stem orientation

  - Instrument Air

- Pipe Material:

  - ≤ 4”:	Galvanized carbon steel

  - > 4”:	Stainless Steel

- Size of Branch

- Potential Consumers:

  - Reciprocating compressors with unloading valves

  - Filters for particle removal from gas stream may need instrument air for self-cleaning

  - Package (Bagging, palletizing, …)

  - Probable consumption for panels and electromotors

  - FFG Panel of Flare Package.

  - Analyzers/Analyzer House

  - Remote operated fire monitors

  - UV Scanners in fired heaters (Typically ~8 Nm3/h for each scanner)

  - DM Water

Provide DM connection for the following consumers:

- Reciprocating compressors which required water cooling (Machinery cooling to be discussed.)

- Surface condensers

- Pump seal flush

- Condensate flash drums with condenser at top where they are equipped with a water seal (Preferred)

  - LPS

- Snuffing Steam: For the quick estimation of snuffing steam line size, 7.1 kg/Hr/m3 of the radiation section can be used.

  - MP/HP Steam

- Isolation at Battery Limit

- Check if flare package is in our scope of work. If so, make sure the steam consumption and headers for smokeless operation of the flare is considered.

- Usually, MPS is used for soot blowers. Soot blowers are necessary where fuel oil is used. When no vendor data is available, consider 5,000 kg/h of MPS consumption for soot blower. Note that soot blowers will NOT operate simultaneously.

- MPS is sometimes used to blow out the fluid remaining in the CDU charge heaters, in case of emergency. A PSLL on process side of the heater may activate the interlock.

Also, MPS may be used to ensure a minimum velocity is heaters of heavy hydrocarbons (e.g. VDU charge heater and Coker Heater). If steam is distributed to each pass by control valve, it is recommended to have a pressure transmitter at inlet to each pass (Downstream of the control valve).

- Where heavy fuel oil (n>40 cSt) is used as heater fuels, atomizing steam should be provided (To be confirmed by vendor). (According to some references the required atomizing steam is 25~50 wt.% of the consumed fuel oil. However, for sizing the steam lines, 100 wt.% of fuel oil should be used.)

  - Steam Condensate

- Gathering of hot condensates with different pressures (Condensate recovery)

- Hot condensate (Contaminated and non-contaminated)

- Cold Condensate

- Pumped Condensate

- Atmospheric Steam Condensers

- Intermittent steam heaters

  - Nitrogen

- Watch the purity!

- Nitrogen may be used as back-up to instrument air (and remove the high-pressure IA compressor and drum).

Important Note: In case instrument air is used for analyzers as reference gas, the nitrogen back-up may cause analyzer malfunction.

  - Utility Stations

- Plant Air, Plant Water, LP steam are available in oil/refineries. Nitrogen should be available where required.

- Plant air should be available at all manholes and tower platforms

  - Ensure nitrogen or instrument air is available for bubble type level transmitter

  - Bidirectional flowmeters for utility headers (ISO 5167-2 Ed.2022): Only one plate with two FTs (DPT) can be used to measure the reverse flow as well as normal flow.

  - Shall we consider sample connection on utility lines at battery limit?

  - Consider blind flanges for flushing during pre-commissioning.

  - Buildings:

| Building (1) | Plant Air | Inst. Air | N2 (2) | Potable Water | Plant Water | LPS (4 barg) | Fuel Gas (3) | Fire Water |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Laboratory (3) (6) |  |  |  |  |  |  |  |  |
| Workshop |  |  | (4) |  |  |  |  |  |
| Control Building |  |  |  |  |  |  |  |  |
| Substation |  |  |  |  |  |  |  |  |
| Analyzer House (5) |  |  |  |  |  |  |  |  |
| Restaurant |  |  |  |  |  |  |  |  |

    - Ask HVAC engineer if utilities are required in buildings for HVAC purposes.

    - Where nitrogen is supplied in buildings, the nitrogen valve shall have different color and shall be easily recognizable.

    - Where fuel gas is used in buildings, it should be low pressure natural gas. Blending with process off-gas is not acceptable.

    - Cooling water and LPS may be required for some petrochemical plants.

    - In case sink is available in analyzer house, potable water is required. Most often, prefabricated analyzer houses have no sinks.

    - Consult laboratory specialist

# Locking Device and Car Seal Device

  - Introduction

  - PSVs (Isolating valves and valve on remote sensing line of pilot operated valves)

Note: The isolating valve of spare PSVs at their outlet shall be locked open.

  - Isolating valves of Auto-start Pumps (e.g., fired heater feeding pumps) should have locking device.

Note: Sundyne pumps shall start with open discharge valve

  - Isolating valve of the closed drain header at inlet to the closed drain drum.

  - Isolating valve of closed drain drum to flare header

  - Isolating valves of Depressurizing Valves (BDVs)

  - Isolating valve at downstream of ARCVs (on minimum flow line).

  - Less Convenient Cases:

- Isolation valve of PT (from equipment/pipe) which initiate an interlock in case of high pressure.

- Isolating valve on roof drain of atmospheric storage tanks with external float roofs should be CSO.

# Steam Trap and Condensate Pot

  - Steam Trap Types (Thermostatic, Thermodynamic and Float Type)

For type selection, refer to DEP 31.38.01.11 (Section 5.4).

According to this DEP:

“A check valve shall also be installed downstream of any trap when condensate will be discharged to an overhead return line or against a lift. The pressure due to the lift shall be added to the pressure in the overhead return line when determining the total back pressure against which the trap discharges”, but this is not recommended unless for:

- Intermittently operating heat exchangers

- When condensate of two steam headers with different pressures are collected into one common condensate header. In this case, NRV can be considered on condensate from lower pressure source.

Remember that thermodynamic steam traps can act as a check valve.

  - Consider installed spare for process steam traps

  - Bypass valve for steam traps

- Generally, bypass valve is not required unless otherwise specified in project criteria.

- Provide installed spare for process steam traps.

  - When to use condensate Pot:

- High Steam Flow (e.g., > 4,000 kg/hr)

- Heat exchangers with inlet steam supply pressures of 345 kPa(g) (50 psig) or less, with or without inlet steam throttling, that discharge into a closed condensate system

  - Pump Trap:

- High backpressure in comparison to the steam pressure at steam heater inlet (e.g., control valve on LPS)

- Intermittent steam heater

  - Different schemes for steam heaters (including reboilers)

    - Control valve on Steam Line

2. Control Valve on Condensate Line

# Pipe Specification Break

  - Having the same pipe rating should not be misinterpreted. A new line number is required when the pipe design condition can vary (e.g., downstream of the control valve assembly) or when a new piping class is to be specified. (See IPS-E-PR-230 §8.5.). For example, the line from the suction block valve of a centrifugal pump should have different line number to the main (common) suction line even if the lines rating is the same (except the valve is LO/CSO or a PSV is provided. See class break around centrifugal pumps for more information.

  - Control Valves

- Rating of Control Valve Flanges shall be minimum 300#. It is not required to show the class break but ensure piping department will consider the issue.

- Be careful of choked flow!

  - Blow Down Valves (BDV)

  - Sometimes project specification calls for a minimum rating (300#) for small nozzles (e.g., equal or lower than 1-½ inches). Ensure piping engineers are aware of this issue.

  - Steam-out Connection (IPS vs UOP/JGC)

  - Steam Traps

  - Material Change (e.g., CS-SS)

  - Around PSVs

- Minimum 300# for inlet nozzle

- Spare PSVs

- API 526 (API 526 may be used but note that it is a user standard)

  - Around centrifugal pumps

- Suction Valve

  - Auto-start pumps with CSO valve

- On-Off Emergency Isolation Valve

- B31.3: See 302.2.4.f.1 (a & b)

- Pumps with high pressure rise

  - Design consideration for Line Class Break at high pressure pump

In case line class 900 # and higher, consider providing a relief valve at suction of pump, so that lower line class could be selected. Since pump mechanical seal section could not withstand for significant high pressure (generally over 50 barG, depended on pump vendor), some protection device is required for shut off operation. A provision of relief valve could cover both mechanical seal and shut

off condition during warm-up operation. Line class could be selected as same as suction line if a relief valve is provided. This consideration will provide less expensive facilities, totally.

A pressure relief valve [or LO/CSO suction valve] shall be provided to protect mechanical seal, if the pump can be exposed pressure from the discharge of the associated operation pump, or from the external flushing supply source, while pump is idle with its suction valve closed. The set pressure of this relief valve should be as same as design pressure of the suction piping and less than maximum static pressure of the mechanical seals.

  - Using PSV at suction line

  - Using LO isolating valve at suction and DBB at discharge

  - For some pump seal plans (e.g., seal plan 52), normal operating pressure of seal drum is low. But in case of seal failure, process fluid will enter the seal drum and increase the seal pressure.

  - Around Heat Exchangers (Double-pipes excluded): 10/13 rule (e.g., Water Coolers)

  - Be careful of choked flow (Gas phase)!

# Insulation Kit and Insulation Flange

  - Insulation Kits are designed to combat the effects of corrosion often found in flanged pipe system. Galvanic corrosion between dissimilar metal flanges, flange insulation associated with cathodic protection of underground piping are other examples where flange insulation Kit Gasket can be put to effective use.

  - Dissimilar Piping material: Insulation flange may be required. Consult material specialist

  - Underground Lines connected to above ground equipment may need insulation flange to avoid electrical conduction between facilities which are under cathodic protection and the above ground facilities e.g.:

  - Cooling Water lines

  - Closed Drain (Connection to process equipment and closed drain drum)

Underground lines not connected to above ground facilities do not need insulation flange.

  - Large Steel Storage Tanks: Insulation kits may be required. Consult cathodic protection specialists. See API 651 (2024); § 5.1.4 & § 7.3.6.1)

# Steam Condensate Drip Legs

  - Size

Generally, the size of drip legs is a piping issue. The following criteria, which is applied by Snamprogetti in BAORC, is just for information.

  - According to IPS-E-PI-240, the size of drip pots is as follows:

| Main Size | Pot Size |
| --- | --- |
| NPS 4 and below | Main Line Size |
| 6" | 4" |
| NPS 8 and above | 6" |

  - Location: The final decision should be made based on 3D model.

- Usually considered at

  - Upstream of Steam Turbine

  - Upstream of the Unit battery limit main block valves (Piping issue)

  - Upstream of Vertical Upward Run of line (Piping issue)

  - Dead sections e.g., at the end of headers and subheaders (Piping issue)

  - Upstream of isolation valve on snuffing steam line

  - At low/medium pressures steam out of back pressure turbines

  - Upstream of control valves in steam service

- Might be required at

  - Upstream of control valve on stripping steam line

  - Downstream of desuperheaters

# Line Size

  - Check the size of the following lines as soon as possible:

- Pump suction lines

- Line from/to reboilers or charge heaters

- Tower overhead lines

- Lines which shall be sized based on the marginal available pressure drop

  - Buried underground lines

- Pressurized (Minimum 2”)

- Non-pressurized (Minimum 4”)

  - Closed Drain Headers (2” for branches, 4” or 6” for sub-headers, 6”~8” for main headers)

  - Mechanical Strength

- Remote sensing pilot operated PSVs (Minimum 1”)

- Nitrogen used for level measurement of a lethal or pyrophoric fluid (Bubble type): The required tube length shall be minimized. The pipe diameter shall be minimum 1”

- Hydrogen Lines (Minimum 1”)

- Depressurizing Lines (Usually≥ 2”)

  - Minimum allowable line size (Use minimum ½” for utilities and ¾”  for process lines unless otherwise dictated by licensor).

  - Drain lines from machinery shall be ¾” or larger.

  - Tank Over Flow line shall be at least one size larger than the largest liquid inlet line

  - Air-cooler Manifold

- Attention: The allowable pressure drop in condenser datasheet usually indicates the total pressure drop in from tower overhead nozzle to the accumulator.

As a basic rule, it is recommended that the pressure drop across manifold piping (ΔPm) does not exceed about 25% of the total pressure drop (ΔPm+ΔPCond.).

- Check if the inlet/out stream to/from the condenser is two-phase.

- Check the pressure drop for each bundle for single phase inlet/outlet.

- The hot bypass flow must meet condensation by heat release through the receiver wall and condensation in the receiver roughly corresponds 5-10% of bypass flow. The criterion mentioned for hot by-pass line in FW STD (507) is very conservative and results in oversizing the hot by-pass line.

  - Utility lines for spare equipment (e.g., cooling water to surface condensers, pumps and compressors)

  - Process lines for M out of N equipment: Assume the working equipment are at the latest ones connected to header (e.g., 2 out of 3).

  - Minimum size for utility main header should be 2 inches.

  - When a heat exchanger has a bypass for on-stream maintenance, the bypass line size may be one size smaller than the main line size

  - The piping pressure drop from the pressure measurement point to the burner should be designed for no more than 1/2 psi (3.4 kPa) at maximum heat release for all anticipated fuel gas compositions.

  - Is following the design criteria a must?

  - Check the following line sizes in early stages of project:

- Tower overhead lines

- Thermosiphon lines

- Cooling water main header

- Lines with high design temperature or large diameters (NPS>8 and DT>250°C)

- Two phase flow lines

  - Pipe Steam-out

Where steam-out is required for a line, the steam-our connections may be as follows:

- Line Size ≤ 10”		S/O connection: 1”

- 10” <Line Size ≤ 20		S/O connection: 1 ½”

- 20” <Line Size		S/O connection: 2”

  - Size of flushing oil connection

- Level Instrument		½ ” ~ ¾”

- Pump Suction		1 ½” ~ 3”

Note: When light hydrocarbon used for flushing is anticipated to contaminate bottom products to such an extent that off-spec products are produced or that bottom product cannot be fed to the subsequent unit, “intermittent purge” shall be employed. It means that the part to be flushed should be isolated from the process. In this case, the part to be flushed shall be drain after flushing.

Continuous Purge for Bridle (Stand-pipe)

Intermittent Flushing Arrangement for: Drain connection are necessary, but not shown on the below arrangement.

On-Stream Maintenance (Including Pump Suction)

Intermittent Flushing Arrangement for RFCC: Drain connection are necessary, but not shown on the below arrangement. Note: Spectacle blind (Permanent or Temporary) shall be provided on fuel gas (or N2).

Flushing Oil Header

# Control and Instrumentation Tip Points

  - Signal Types (Analog, Digital, Software, Pneumatic, Hydraulic, Capillary Tubes)

  - Process Override Switches (POS), Maintenance Override Switches (MOS)

- MOS is usually considered for all interlock with below exceptions:

  - Start permissive

  - Emergency Hand switches

  - All trips with 2 out of 3 voting logic or 2 out of 2 Voting

Note that MOS shall not override the alarm function. Also, output shall not be overridden or isolated.

- POS is usually considered for startup conditions

Examples:

- Fuel Gas System

- Pump/Compressor Startup

- Analyzers of Continuous Catalytic Reforming unit

  - Software or hardware reset for interlocks

  - Consider defining CRITICAL alarms with buzzer or hardware lamp in control room.

  - Make a note to specify if an instrument loop is SIL 2 or SIL 3.

  - Split Ranges

- Dead Band

- Using two different controllers to facilitate changing the dead band

  - High/Low Selector Switches

See also the typical P&ID for fuel skid (High selector).

  - The instrument which activates ESD:

- Instrument shall be independent to those instruments used for unit control (FCS/DCS). Two-out-of-Three arrangement can be used for both control (Median) and ESD activation.

- Independent connections required for level instruments

  - Indication of process parameter (Pressure, Temperature, Flow, …) of another unit

  - It is recommended to stop mixers in a drum/tank in case of low liquid level.

  - Pressure instruments

- Diaphragm seal to be shown, if applicable

Diaphragm seals can be an integral part of the pressure instrument or to be connected by capillary tubing. In vibrating service, diaphragm seals should be remotely mounted and have armored stainless steel capillary tubing.

- Pressure gauges are recommended for the following locations:

  - Pump discharge

  - Compressor Suction, Discharge and inter-stage KODs (Do not duplicate)

  - Many compressors suction lines and suction line of heavy-duty pumps

  - Top of Vessels and columns

  - Bottom of fractionators (Vapor Space)

  - Non-indicating pressure transmitters (To be discussed)

  - The sensing side of each direct-operated pressure regulator and field pressure controller

  - Instrument air reservoir for valves (These reservoirs shall have both low-pressure switch (PSL) and local pressure gauge).

- Differential Pressure Transmitter (DPT) with High Differential Pressure Alarm (DPAH) is required/recommended to sense the pressure difference between:

  - Hot and cold side of plate heat exchangers (Necessary for high duty exchangers)

  - Atomizing steam and fuel oil

  - Upstream and Downstream of Filters

  - Inlet and outlet of reactors, adsorbers and packed columns

  - It is recommended for pressure drop across the strainer for heavy duty pumps. It is required for steam generation circulation pumps (e.g., in HRSGs).

  - Coalescers (Preferred)

- Pressure instrument to be considered for double diaphragm metering pumps. Instrument to be supplied by pump vendor.

- Where piping length for DPT is high and fluid is pyrophoric or very toxic, using two different PTs may be justified.

- Pressure switch/transmitter shall be considered between rupture disk and downstream PSV. If the system is operating under vacuum, Low Pressure Alarm (PAL) shall be considered. Otherwise, High Pressure Alarm (PAH) is required.

  - Temperature Instruments

- Temperature transmitters (TT) are recommended for the following locations:

  - Columns:

    - All inlet and outlet lines:

- The TT on the bottom product line from tower should be equipped with TAL and TAH.

- Process inlet and outlet of heat exchangers with automatic temperature/duty control.

    - The upper tray if stripping steam is used in the column

  - Air coolers:

    - At outlet of air coolers: In case the process outlet temperature is controlled automatically and it is an important operating parameter, a temperature transmitter, in addition to the one in the closed loop, should be provided with high/low alarm as required.

    - Outlet of all air-coolers (common outlet line) with TAH (and TAL if freezing/congealing is a concern).

  - Refer to recommended instruments at battery limit (this chapter), Instrument for compressors (this chapter) and fired heaters (§10.6).

  - Reactor Inlet and outlet lines

  - Adsorption bed/Molecular Sieve inlet lines

  - Critical temperature controllers and transmitter

    - Additional temperature transmitter to be considered separate from the controlling loop

  - Orifice flow meters

    - Consider a TT for gas/vapor services which temperature compensation is required. The PT should be installed upstream of the orifice (Close to the flowmeter).

    - Check if a TT is required for heavy hydrocarbons to correct viscosity

- Temperature Gauges are recommended for the following locations:

  - Outlet of water coolers (CWR Line)

  - Inlet/outlet of heat exchangers

  - Reservoir for Lubrication oil of mills, compressors, etc.

  - At battery limit on Process or utility lines with temperature different to ambient.

  - TW (TG or TT for fouling services) shall be provided on each bundle outlet of air coolers.

  - Flow Measurement Instruments

- Flow Measurement is recommended for:

  - Feed, Product and by-product, Reflux, Stripping steam, reboiler heating medium, BFW to Steam generator, etc.

  - Meter Runs

  - Eccentric orifice to be used in fouling services

Consider using other types of flowmeters (e.g., turbine flowmeters) instead of orifice when the fluid is fouling or congealing such as heavy hydrocarbons. A strainer is necessary upstream of the flowmeter.

  - Sometimes, a receiver gauge is required for reading at a certain place in field. In this case, use the following representation and take confirmation of I&C specialist:

  - HART Transmitter:	Take signal from transmitter

  - FF Transmitter:	Take signal from system

  - Control Valves and On-Off Valves

- Internal leakage: See FCI 70-2 or IEC 60534-4

  - Generally, leakage class is not a concern for control valves without on-off action. Class IV is achievable for most valve types. Lower class is also acceptable in many cases.

  - Tight shut-off means class V or class VI. However, using class VI (Soft seat) should be minimized as they are more susceptible to wear and tear. Use class V where tight shut-off is required for on-off valves or control valves with on-off action.

- Block and By-pass Valve (Type and Size)

  - IPS Concept and its problems (IPS-C-IN-160)

    - § 6.5.: The by-pass valve should be one size larger than the control valve but may, by specific agreement, be the same size as the control valve.

    - § 6.11.: The CV factor of the by-pass valve shall be at least equal to and not more than twice the CV factor of the control valve. The by-pass valve shall be a globe valve for sizes 4 inches and smaller, and a gate valve for sizes 6 inches and larger.

  - Parallel control valves: Provide one bypass for the largest valve

  - Some control valves in gas or flashing services may have different inlet/outlet line sizes

  - NIOEC Concept for size of block valves

  - When there is a bypass for a TSO control valve which is normally closed, there should be gate valve is series to the bypass globe valve.

- Bypass valve is not required for:

  - Temporary services at which the control valve can be dismantled and fixed without interruption of operation

  - On-Off Valve

  - Compressor Anti surge valve

  - PCVs

  - Pump Minimum Flow Line Control Valve

  - Control Valves larger than 12” (Handwheel may be used)

Note: Bypass is required for back pressure regulator on chemical drums if the PCV is not sized for maximum inlet liquid flowrate.

- Consider a dead band for set pressure of PCVs (Minimum ±10%) where arranged like above figure.

- Parallel PCVs should not have similar set pressures:

  - Back pressure regulators: Usually, the larger PCV has the highest set pressure

  - Downstream pressure regulators: Usually, the larger PCV has the lowest set pressure

- Instrument Air Reservoir Drum for valves (PI, PSL and NRV)

- Requirement of tight shut-off (i.e., class V or VI) and hand-wheel should be shown on P&ID (e.g., TSO).

- Failure Position

Typical Failure Positions:

  - Process Side of Furnaces:					*	FO

  - Fuel Side of Furnaces:						FC

  - Pump Minimum Flow and Compressor Anti-surge:		FO

  - Compressor Spill back/Blow off:					FO

  - Compressor isolating valves (used for centrifugal compressors with power of >750 kW):							FC

  - Valves discharging to flare (except depressurizing valves):	FC

  - BDV (Depressurizing Valve):				**	FO

  - Unit feed/product:							FC

  - Push-Pull Systems							FC

  - Column Reflux							FO

  - BFW to desuperheaters:					**	FC

  - Level Control Valves:						FC

  - Pressure control valve on lines from column receiver		FC

  - Hot by-pass valve on tower pressure control loop:		FC

Main valve on the overhead line (upstream or downstream of the condenser) 	FO

  - Dampers on air or flue gas					***	FO/FL

  - Louvers of air-coolers					****	FO

  - PDV on (MP) Steam atomizing line to fired heater:	*****	FC

  - FV on Velocity Steam						FO

* FC with minimum mechanical stop may be acceptable. See API556 §3.2.3.2

** Arguable. To be decided case by case.

*** Where ID fan is provided, the damper should be FO. See API 556

**** When there is a risk a freezing or congealing, FC may be selected.

***** If the valve is FC, there should be a PSLL downstream of the valve to cut-off the fuel oil in case of low low pressure of steam. This is the preferred solution. Another alternative is to consider FO for the valve (No PSLL required).

- Definition of FL(O) and FL(C)

If a Fail Last control valve is used, the spring/flow direction should be shown on P&ID (FLO/FLC)

  - FL/DO and FL/DC

- Limit Switch(es) should be provided for the following cases:

  - On-off valves including SDVs

  - Valves discharging to flare/safe location which are expected to be close during normal operation (Recommended not mandatory)

  - Valves which their position is start-allowance (e.g., anti-surge valve of a centrifugal compressor)

  - Valves which their position activate an interlock (e.g., on-off valve at upstream of a pump)

  - Emergency Push buttons for isolation of Compressors/liquid containing drum or shut down of Fired Heaters

  - I/P for automatic adjustable stroke pumps. Failure position should be specified as well (i.e., whether stroke is null or maximum in case of instrument air failure).

  - Transmitters (and their connection) activating a safety interlock should be independent to the transmitters sending signals to the control system.

A possible exception to this rule is when voting is provided (see below). In case of using 2oo3 voting, the average median signal can be used for control.

  - Voting

The intention is to increase availability, reliability or both. The following table represents the availability and reliability in comparison to single transmitter (or switch):

| Voting | Availability | Reliability |
| --- | --- | --- |
| 1oo2 | ↓ | ↑ |
| 2oo2 | ↑ | ↓ |
| 2oo3 | ↑ | ↑ |

  - Mechanical or DCS Min/Max Stop, if required, shall be specified on P&ID (especially when related to safety). Here below are some examples for possible min/max stop requirement.

| Process Side of Fired Heaters | Min Mechanical Stop |
| --- | --- |
| Damper of Fired Heater Stack | Min Mechanical Stop |
| Steam to hydrogen reformer | Min Mechanical Stop |
| Atomizing Steam for fuel oil | Min Mechanical Stop |
| Valves with gas blow-by scenario | Max Mechanical Stop* |
| Inlet throttle control valves of centrifugal compressors or Blowers or blowers | Min Mechanical Stop |
| Valves on Fuel Gas to Main Burners | Max Mechanical Stop* |

* Might be required if the relief load is too high

** Might be required to avoid burner flame-out and tube failure, if high high pressure of fuel gas to heater does not trip the heater.

  - Delay required for:

- Pump Trip (Auto start Configuration)

- UV Shut Down at suction/discharge of Pumps/Compressors

- Trip of fired heater in case of low low flow of process side.

  - XVs on liquid services at battery limit may need to close slowly. If this is the case, it shall be noted in documents.

  - Cascade Control (Temperature-Flow, Level-Flow)

  - Trip or Stop?

  - When an analyzer should be retractable, put a note on P&ID.

  - Hydraulic signals and hydraulic oil return line

  - Failure position of louvers and adjustable pitch fans to be specified on P&IDs.

  - Is remote start required for rotary equipment?

- Air-coolers

- Pumps

- Compressors

  - Drain of Level Instruments

  - Switch or Transmitter?

  - Piping/tubing for DP transmitter

- Free Draining

- Toxic Services

  - Recommended Instruments at battery limit

| Item | Service | Minimum Recommended Instruments |
| --- | --- | --- |
|  | Unit Feed/Product | PG, PT, TI, TT, FQI, SC for all process streams to/from unit (e.g., Hydrogen); P/T Compensation as required |
|  | Steam | PG, PT, TI, TT, FQI (Orifice), P/T compensation to be considered. In case a utility is only used in utility stations (e.g., LPS), no instrumentation but one PG is required at BL. |
|  | Hot Condensate (Two Phase) | PI, TI |
|  | Flashed Condensate (Single Phase) | PI, PT, TI, TT, FQI |
|  | CWS | PI, TI, TT, |
|  | CWR | PI, TI, SC, FI |
|  | Plant Water | PI |
|  | Potable Water | PI, TI |
|  | BFW | PI, PT, TI, TT (TAL?), FQI, SC (Only if BFW is supplied by others) |
|  | Demin Water | PI, PT, FQI, SC |
|  | N2 | PI, PT (PAL if N2 is used for critical services such as compressor seal system), FQI, SC |
|  | Plant Air | PI, If plant air has a process consumer (e.g., it is used as regeneration air) then PT with low pressure alarm and FI is recommended. Evaluate the need for pressure compensation for flow measurement. |
|  | Instrument Air | PI, PT (PAL), FQI In case instrument air is supplied from another complex, a filter/strainer with DPT (DPAH) is recommended at line entry to complex. A by-pass for the filter/strainer is recommended. |
|  | Fuel Gas | PI, PT (PAL), FQI |
|  | Flare | PG or PT FI (External Ultrasonic flow meter) if required by client |

  - Instrumentation for Compressors

- Provide TSV on cooling water return lines.

- The minimum required instrumentation on process side is mentioned in the below table (For more information, refer to Exxon IP 15-1-2).

| equipment | Instrument (*) | Functional Hardware |
| --- | --- | --- |
| Suction K.O. Drum | PG PT LG LT (2oo2 or 2oo3) | To control the pressure via spill back (Recip. Comp.) and indicate the pressure LAHH Compressor Shut-Down LAH |
| Each Stage Suction | TG (VF) (***) TT (VF) PT (**) (VF) | TAHH Compressor Shut-Down TAH PAL PALL Compressor Shut-Down |
| Each Stage Discharge | TG (VF) (***) TT (VF) PG (VF) (***) PT (VF) | TAHH Compressor Shut-Down TAH PAH |

(*)	When compressors are working in parallel, care should be taken to avoid unnecessary duplication of instruments.

(**)	Downstream of strainer

(***) Optional

# Pressure Safety/Relief Valves

  - Weep Holes and drains

  - Where PSV has isolating valves, provide a vent downstream of the PSVs and a drain upstream of the PSVs

  - Isolating Valve

- Is it allowed for PSVs installed on steam drum?

- PSV isolating valves shall have Car Seal device or locking device

- Open/Close arrangement of isolating valves for spare PSVs

  - PSVs venting hydrocarbons to atmosphere

  - Common header

  - By-pass line

- It is required for closed steam-out.

- Required for gas service with at least one scenario other than external fire

- Generally, not required on liquid service unless used for venting purpose

- In case the equipment can be connected to the flare header via another connection (e.g., bypass line of a control valve to flare), the PSV bypass is not required.

- Consider only one by-pass line for multiple PSVs (whether they are spare or not)

- Size of by-pass line (To be discussed)

  - Put a note to inform piping department to design the tail pipe and PSV discharge line for liquid ingress, if there is any relief scenario at which liquid may be discharged from PSV (Recommended).

  - It is recommended (Not mandatory) to consider a straight length at least 10 times of pipe diameter upstream of PSV, if installed on a pipe (See API 520-II; 2020; §4.3).

  - Rupture disk may be installed in series with a PSV for following cases:

- Vacuum Services

- Clogging services such as butadiene or styrene

Consider a PG and a PSH (PSL for vacuum services) between rupture disk and PSV.

NOTE: Rupture disks upstream of PSV (or flame arrestor) shall be non-fragmenting type.

  - If a spare PSV is installed for staggered PSVs, its set point should be equal to the lowest set pressure of PSVs.

  - In the fouling services (e.g., RFCC sour water stripper) the PSV on distillation tower shall be installed at top of the highest liquid level at the column sump, if the service is fouling.

  - TSVs

- TSV is required:

  - For sections of pipe which are steam traced (or there is any other heat source) and contain liquid and may be blocked in. (Do not send the heavy hydrocarbons from TSV to funnels. Use temporary bucket.)

  - For blocked in sections of pipe which contain cold liquid and may develop excessive pressure under ambient temperature. For instance:

    - Intermittent lines e.g., alternative feed/product lines

    - Spare filters

    - Intermittent water injection line

  - For blocked in sections of pipe which may be subjected to excessive pressure due to rays of sun when exposed length of line is more 30 meters or the liquid volume is more than 0.9 m3.

  - Cold side of Heat Exchangers if it can be isolated while the hot side is flowing. Water coolers may be included in packages. The surface condensers remain in this category.

  - Interconnecting Lines which handle liquid

- Do not size TSVs (unless for pipelines or long interconnecting lines)

  - PSV inlet line size

- Chattering

- Rated Flow

- Rupture disk at PSV inlet line

- Number of PSVs

- Shall we follow the 3% criteria for fire case?

- Pilot operated valve with remote sensing lines

  - PSV location

- Top of vessel or vapor outlet line?

- Demister Pad

- Restriction Orifice, Flow meter, any piping element cause pressure drop

- PSVs installed for protection of filter can be installed either on filter body (Recommended) or on the inlet line. Installing the PSV at outlet line may not be a good choice as the dirty filter may not be protected properly.

  - If one spare PSV is considered for a series of staggered PSVs, the set pressure of spare PSV should be equal to the lowest set pressure of operating PSVs.

  - Introduction to PSV types (Conventional, Bellows and Pilot Type)

- Back Pressure

- Pressure loss in PSV inlet line

- The difference between operating pressure and design pressure

- Advantages/Disadvantages of using remote sensing pilot PSVs (e.g., for manual blow down)

- High capacities

# Flowmeters

  - Flow meters in liquid services shall be full of liquid

- Slope or upward line downstream of flow meter is required.

  - Required Straight Run upstream and downstream of flowmeters

- It is not required to specify required straight run upstream and downstream of flow meters but be careful that changing in line size may cause problems for piping engineers. Cooling water header is a good example for this.  Rethink about requirement of by-pass line for flowmeter on large NPS.

- Mass flow meters do not require straight run

  - By-pass for flowmeters

# Smart Software for Preparing P&ID (e.g., AVEVA)

  - Be careful of equipment/instrument with spare in warehouse. The equipment/instrument list will not include such spares.

  - Equipment data and items must be entered as per CADWorx/AVEVA instruction only.

# Hold Management

Typical HOLD Items:

- Compressor discharge temperature

- Pump Flow Rate For pumps with restriction orifice on min flow line

- Vessel dimensions where there is a coalescer inside the vessel

- Vessel dimensions when there are adsorbents/catalysts inside the vessel (*)

- Tower dimensions (shall be confirmed by tray/packing vendor)

- Control Valve, its isolation/bypass valve size and number of expanders/reducers

- PSV orifice size, its inlet/outlet line size & corresponding vessel nozzle size (**)

- Flowmeter connection size

- Pump suction/discharge nozzle size

- Pump block valves, check valve and strainer size (***)

- Pump minimum flow line arrangement/requirement

- Details of Pump Vent & Drain

- Tag number of specialty item.

- Package details within vendor battery limit

- Line size

- Elevation

- Interconnecting/reference document number

- Required utility and line sizes for packages, pump or compressor

- Type of heat exchanger & its nozzle details

- Number of sensors

| * | Unless the vessel size is provided by the catalyst supplier |
| --- | --- |
| ** | Please note that API 520 is a user standard not a vendor standard |
| *** | Unless there is another criterion: - the size of strainer and pump suction block valve should be same as the suction line. - the size of check valve(s) and discharge block valve should be same as the discharge line. It is recommended to deviate such criteria sometimes requested in contracts after receiving client confirmation |

# Pickling

  - Mill scale, dirt, heavy greases, and other foreign materials that could enter the compressor and result in operating and maintenance problems must be removed from the compressor system as required in the Piping & Instrumentation Diagrams (P & IDs).

  - The following items must be acid cleaned:

- Make-up compressor suction lines Suction drums may be acid cleaned where practical. It is mandatory for reciprocating compressors and recommended for centrifugal compressors. Chemical cleaning connections (normally 2” blank-off flange) should be provided for the precommissioning operation of reciprocating compressor: One at minimum distance from outlet of K.O. drum nozzle and one at minimum distance from compressor flange

- Fuel gas lines to gas turbine burners.

- Oxygen service Lines

- Halogen (F2 and Cl2 Service)

- Remote sensing line of PSVs (Preferred).

  - Equipment and Lines which need pickling should be specified on P&ID (IFC revision).

# Miscellaneous

  - Evaluate if a recirculation line should be considered for start-up so that the off-specification product(s) can be recirculated to the unit

  - Check whether sample connection is considered at:

- Upstream and downstream of adsorbers/reactors

- Any point an analyzer is installed (and laboratory test can be conducted).

  - Items which need to be highlighted for safety department. The final decision will be taken by safety department.

- F&G Department: Consider hydrocarbon detector for below cases

  - Air inlet to compressors or blowers with atmospheric air inlet (This is to alert operator in case of HC release)

  - Any uncovered sump within the process area (e.g., closed drain sump)

  - Bellows type PSVs if NOT installed in safe location. The alternative solution is to send the PSV vent to safe location via tubing/piping.

  - Any detector is required as per P&ID (Usually mentioned in NOTES).

Inform if any detector is required as per P&ID (Usually mentioned as NOTES).

  - Electrical Department:

  - Earthing connections required for trucks or IBCs. Specify whether “Earthing Switch Low” is necessary.

  - Lighting for sight glasses

  - Any equipment/line/instrument which need electrical tracing. Note that sometimes only part of line may need tracing and this may not be mentioned in line list.

  - CCTV may be required for:

    - Truck loading/unloading area

    - PSVs which discharge hydrogen or hydrocarbon to atmosphere

    - Flare monitoring

    - Process Concerns

  - Specify if hoses need to be dry-break or ball valves need to be dry disconnect.

  - Isolation valves of packages shall NOT be provided by vendor.

  - Level Instrument (Location and protection)

# Typical Check List

| Item | P&ID Sheet | 1 | 2 |
| --- | --- | --- | --- |
| 1* | Title Block |  |  |
| 2* | Client Comment |  |  |
| 3* | Piping Comments |  |  |
| 4* | Instrument Comments |  |  |
| 5* | Notes |  |  |
| 6* | Holds |  |  |
| 7* | Spell Check |  |  |
| 8 | Equipment Data (Including Size of Nozzles for Fixed Equipment) |  |  |
| 9 | Instrument Type (Including Level Transmitters) |  |  |
| 10 | Isolation Philosophy |  |  |
| 11 | Level Instrument Vent and Drain |  |  |
| 12 | Line Slope and Free Draining Req. |  |  |
| 13 | Lines/Signals from/to other Sheets |  |  |
| 14 | Pipe Spec Break |  |  |
| 15 | Tracing |  |  |
| 16 | Line Finishing |  |  |
| 17 | Size of Vent/Drain/Steam Out on Vessels + Size of Vent/Drain on Lines |  |  |
| 18 | Checking Signals (Remote/Local Stop/Start) for rotary equipment |  |  |
| 19 | Failure Position /HW /TSO of Control Valves |  |  |
| 20 | Checking of Process Data Sheet of all equipment with P&ID |  |  |
| 21 | Checking of Mechanical data sheet of equipment with P&ID |  |  |
| 22 | Consistency of package, pump or compressor vendor data with P&ID |  |  |
| 23 | Flare/Water Connection for SCs |  |  |
| 24 | Local Push Button for ESD valves (located at safe area) |  |  |
| 25 | Diaphragm seal for PI/PT |  |  |
| 26 | Cause and Effect |  |  |
| 27 | Equipment Elevation |  |  |
| 28 | POS |  |  |
| 29 | Free Draining, Slope: Requirement and Symbol |  |  |
| 30 | Type of reducers (Concentric, Bottom Flat or Top Flat) |  |  |
| 31 | Type of Control Valves |  |  |
| 32 | Size of PSV and Its Inlet/Outlet line |  |  |
| 33 | Control Valve Size |  |  |
| 34 | Alarm List |  |  |
| 35 | Consistency with PFD |  |  |
| 36 | Tie-in Numbering for Packages and Equipment |  |  |
| 37 | Equipment Trim Number |  |  |

* Item need to be checked for each revision.

DRIP LEG UPSTREAM OF COMPRESSOR

(Applicable when suction line is connected from bottom)

FLARE HEADER PURGING ARRANGEMENT (SWEEP GAS CONNECTION)

FULL SYMMETRICAL AIR-COOLER HEADER ARRANGEMENT

RO

RO

RO

RO

RO

RO

If wash water is required and the inlet manifold is not full symmetrical (single phase), wash water should be injected as depicted above. Strainer should be installed upstream of the restriction orifices.

TYPICAL BRANCH TABLE

BRANCH CONNECTION TABLE

| TS | Socket Welding Tee | TW | Buttwelding Tee |
| --- | --- | --- | --- |
| SO | Sockolet | TEE+RED | Tee + Reducer |

Note: Following branch table while preparing/developing a P&ID is not necessary. Piping engineer may follow the branch table for modelling purpose, only.

TYPICAL DETAIL OF STEAM CONDENSATE RETURN PIPING

Symbols as per ISA 5.1.

Table 5.1.1. Instrumentation device and function symbols

Note: Numbers in parentheses refer to explanatory notes in Clause 5.3.1.

| Shared display, Shared control (1) |  | D | Location & accessibility (6) |
| --- | --- | --- | --- |
| A | B |  |  |
| Primary Choice or Basic Process Control System (2) | Alternate Choice or Safety Instrumented System (3) | Discrete (5) |  |
|  |  |  | - Located in field. - Not panel, cabinet, or console mounted. - Visible at field location. - Normally operator accessible. |
|  |  |  | - Located in or on front of central or main panel or console. - Visible on front of panel or on video display. - Normally operator accessible at panel front or console. |
|  |  |  | - Located in or on front of secondary or local panel or console. - Visible on front of panel or on video display. - Normally operator accessible at panel front or console. |

Typical control valve flow coefficient (Cv)

Typical Valve Flow Coefficient (Cv) for Manual Globe Valve

| Size | ¾” | 1” | 1 ½” | 2” | 3” | 4” | 6” |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cv | 2.4-4.6 | 5.6-8.9 | 19-21 | 29-34 | 117 | 210 | 476 |

Typical Distance between Desuperheater and Temperature Instrument

(By Spirax Sarco)

Use with Care

Difference between Coolant and Outlet Temperature (°C)

Variable Orifice

Single Axial Injection

Difference to Sensing Point (m)

Difference between Coolant and Outlet Temperature (°C)

NOTE: The required minimum distance may vary from vendor to vendor.

CAVITY HOLE OF VALVE

Gate valves and ball valves in liquefied gas service have cavities where liquefied gases may be trapped.

During warming up of these valves, this trapped liquid expands and evaporates and may build up high pressure which could damage the valve. There is no body cavity for butterfly valves, globe valves and check valves. Therefore, such valves are excluded to this cavity hole requirement.

In the closed position, the ball or wedge is pressed against the seat at the low-pressure side. In order to avoid leakage through the ball or wedge, the valve shall be installed such that the drilled hole is at the high-pressure side when the valve is in the closed position. These valves shall be clearly marked “Drilled hole this side” and this position shall be shown on the P&ID clearly. Refer to the typical example for the direction of cavity hole. The requirement is applicable for all the valves including instrument valves.

NOTES:

- Get the consultation from piping group which line classes require the cavity hole.

- In the trunnion type ball valve, there are two methods of pressure release from cavity pressurization, i.e., cavity hole installation or self-relief system. When using the self-relief type of ball valve, care shall be taken to apply the low temperature service because the self-relief ball valve can only release the pressurized cavity fluid to the low-pressure side, this design is usually not acceptable on the process safety point of view.

Typical examples for the direction of cavity hole are shown below:

Drilled hole side

Relief Valve Isolation

To Flare

CAVITY HOLE OF VALVE (continued)

Equipment isolation

Storage

Drain Line

Main Line

NOTE: Also, the direction of installation is important for special types of ball valves (e.g. orbit valve).

SNUFFING STEAM RING AROUND FLANGES

The snuffing steam rings may be used around flanges where the fluid operating temperature is above its auto-ignition temperature.

LPS

Locate this valve at grade and at a location 15 m or more away from the heat exchanger and group it with other valves installed for the same purpose.

Sample P&ID for snuffing steam ring

TYPICAL P&ID FOR FIRED HEATERS

JGC Samples for Fired Heaters

Atomizing Steam

Typical Forced Draft Heater

Typical 1 Balanced Draft Heater

Typical 2 Balanced Draft Heater

Typical Catalytic Reforming Reaction Heater

Typical Fuel Skid for Fired Heater

TYPICAL SHUT DOWN LOGIC FOR FIRED HEATERS (IPS-E-PR-810)

API 560-INSTRUMENTATION FOR FIRED HEATER

(Just for Information)

TYPICAL DIAGRAM FOR DRAINAGE IN TANK FARM

TYPICAL DIAGRAM FOR NEUTRALIZATION